Densities and viscosities for binary mixtures of n-heptane with alcohols at

different temperatures

MARIA MAGDALENA BUDEANU*, VASILE DUMITRESCU

Chemistry Department, Petroleum and Gas University of Ploiesti, Ploiesti, Romania

*Corresponding author. E-mail: m19magda@yahoo.com

Abstract: Densities (ρ) and viscosities (η) of the binary systems n-heptane with alcohols (ethanol, n-propanol and iso-propanol) were measured at temperatures between 288.15 and 308.15 K and atmospheric pressure, over the whole composition range. The excess values of molar volume (VE) and viscosity (ηE) were calculated from experimental measurements. The excess functions of the binary systems were fitted to Redlich-Kister Equation. Comparison between experimental excess molar volume and that calculated from Flory and Prigogine-Flory-Patterson theories has also been made. Viscosity results were fitted to the equations of Grunberg-Nissan, Heric-Brewer, Jouyban-Acree and McAllister. Also, the activation energies of viscous flow have been obtained and their variations with compositions have been discussed.

Keywords: n-heptane; ethanol; n-propanol; iso-propanol; Flory and Prigogine-Flory-Patterson theories.

RUNNING TITLE:  DENSITIES, VISCOSITIES FOR N-HEPTANE + ALCOHOLS

INTRODUCTION

Experimental liquid densities and viscosities of pure hydrocarbons and their mixtures are useful in design and simulation processes. The physicochemical properties play an important role in the understanding of several industrial processes.1 Therefore, experimental measurements are needed to understand the fundamental behaviour of these properties and then to develop new models.2 The derived properties (excess molar volumes and excess viscosities) in combinations with other mixing properties provide valuable information for qualitatively analyzing the molecular interactions between molecules.3

In this work the densities and viscosities of three binary systems of n-heptane + ethanol or + n-propanol or + iso-propanol were measured at temperatures between 288.15 and 308.15 K, over the entire composition range and at atmospheric pressure. The investigations into the literature have shown that these systems have been studied but not quite in the same conditions.

Jimenez et al.4 reported the values of viscosities and densities for n-propanol + n-heptane at 293.15, 298.15, 303.15 and 308.15 K. Keller et al.5 investigated the excess molar volumes of n-propanol + n-heptane at 298.15 K and Tanaka et al.6 studied excess molar volumes of propan-2-ol + n-heptane at the temperature 298.15 K. Pereiro et al.7 reported densities for ethanol + n-heptane at 293.15, 298.15, 303.15 K and Orge et al.8 studied densities and viscosities for ethanol + n-heptane and n-propanol + n-heptane at 298.15 K. Papaioannou et al.9 presented densities for n-heptane + ethanol at 298.15 K and Zeberg-Mikkelsen et al.10 investigated dynamic viscosities for ethanol + n-heptane at 293.15, 313.15, 333.15 and 353.15 K. Zielkiewicz11 reported excess volumes of heptane + propan-2-ol at 313.15K and Rajendran12 studied density and viscosity of binary liquid mixtures of n-heptane with iso-propanol at 298.15K.

From our experimental data, excess molar volumes and excess viscosities were calculated and correlated by the Redlich-Kister Equation. The experimental data of excess volumes of the binary mixtures were used to test to applicability of Flory13-16 and Prigogine-Flory-Patterson17,18 theories, which has not previously been employed for the study of excess molar volume of binary mixtures of ethanol, n-propanol and iso-propanol with n-heptane. The thermodynamic functions of activation of viscous flow have been estimated from the experimental data. The purpose of this paper is also to test five semiempirical equations with one until four parameters to correlate viscosity of binary mixtures.

EXPERIMENTAL

Materials

The chemicals n-heptane (mole fraction purity > 0.990) was supplied by Sigma-Aldrich, n-propanol (mole fraction purity > 0.999) was obtained from Merck, ethanol (mole fraction purity > 0.993) and iso-propanol (mole fraction purity > 0.997) were supplied by Chemical Company. The chemicals were dried over molecular sieves (Fluka type 4 Å). The purity was checked through chromatographic analysis. Samples were determined by weighing at atmospheric pressure and ambient temperature using a Adventurer Pro AV 264CM balance with an uncertainty of and precision of ± 10-4 g. The error in the final mole fraction is estimated to be less than ± 0.0001. Conversion to molar quantities was based on the relative atomic mass table of 2006 issued by I.U.P.A.C.19

Measurements

The densities were determined by hydrostatic weighing method of Kohlrausch20 with the precision of ± 0.05 kg m-3.  The estimated uncertainty for density was ± 0.15 kg m-3. The experimental technique has been previously described21, an ultra-thermostat type U10 (Freital) has been used to maintain a constant temperature (± 0.05 K). The uncertainty in the excess molar volume was estimated to be ± 5.5x10-8 m3.mol-1. Viscosities of the pure compounds and of the binary mixtures were determined with an Ubbelohde kinematic viscometer22, Viscosity Measuring Unit ViscoClock (Schott-Gerate GmbH), that was kept in a vertical position in a water thermostat. A thermostatically controlled bath (U10 constant to

± 0.05 K) was used. The kinematic viscosity was calculated using the relation (1):

ν=At−Bt 	(1)

where t / s is the flowing time of a constant volume liquid through the viscometer capillary. Accuracy of time measurement for the Viscosity Measuring Unit ViscoClock is ± 0.01s. A and B are characteristic constants of the used viscometer, which were determined by taking bidistilled water and benzene (Merck, mole fraction purity > 0.995) as the calibrating liquids for correction of kinetic energy deviations. The dynamic viscosity was determined from the Eq. (2):

η=νρ 	(2)

where ρ is the density of the liquid. The precision of the viscosity was estimated to be ± 0.001 mPa.s. In all determinations, triplicate experiments were performed at each composition and temperature, and the arithmetic mean was taken for the calculations of the viscosity. The uncertainty in the excess viscosity is estimated to be ± 4x10-3 mPa.s.

RESULTS AND DISCUSSION

The measured densities and viscosities of the pure component liquids present good agreement with the literature values, as seen in Table I. Densities and viscosities of the binary mixtures of n-heptane + ethanol, n-heptane + n-propanol and n-heptane + iso-propanol are reported in Table II. The results of this study are in close agreement with the values reported in the literature.

n-Heptane and ethanol densities values reported in the literature differ from our experimental data with a maximum of 0.09%. For n-propanol, densities values published in the literature differ by less than 0.08% and for iso-propanol differ by less than 0.2%.

Viscosity values reported in the literature differ than our results with a maximum 0.8% for n-heptane, with maximum 2% for ethanol, with maximum 1.5% for n-propanol and with 0.6% for iso-propanol.

Figures S-1. – S-6. of the Supplementary material to this work include comparisons of our experimental density and viscosity of n-heptane + ethanol, n-heptane + n-propanol and n-heptane + iso-propanol at 298.15 K with data available in the literature.

TABLE I. Comparison of experimental densities (ρ) and viscosities (η) of pure liquids with literature values at different temperature and atmospheric pressure

| Component | T / K | ρ / kg m-3 |  | η / mPa s |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Experimental | Literature | Experimental | Literature |
| n-Heptane | 288.15 | 688.20 | 687.923 | 0.443 | - |
|  | 293.15 | 684.30 | 683.924 | 0.419 | 0.416928 |
|  | 298.15 | 680.00 | 679.4825 | 0.393 | 0.390027 |
|  | 303.15 | 675.70 | 675.126 | 0.377 | 0.378528 |
|  | 308.15 | 671.40 | 670.927 | 0.358 | 0.347029 |
| Ethanol | 288.15 | 795.80 | - | 1.365 | - |
|  | 293.15 | 789.50 | 789.4530 | 1.221 | 1.198833 |
|  | 298.15 | 785.70 | 785.031 | 1.109 | 1.105031 |
|  | 303.15 | 781.60 | 781.032 | 1.006 | 1.010234 |
|  | 308.15 | 777.00 | 776.732 | 0.922 | 0.903733 |
| n-Propanol | 288.15 | 808.41 | - | 2.491 | - |
|  | 293.15 | 804.40 | 804.2835 | 2.230 | 2.197035 |
|  | 298.15 | 800.71 | 800.2135 | 1.988 | 1.970031 |
|  | 303.15 | 796.71 | 796.4235 | 1.764 | 1.784336 |
|  | 308.15 | 792.90 | 792.2735 | 1.554 | 1.546036 |
| iso-Propanol | 288.15 | 791.20 | - | 2.762 | - |
|  | 293.15 | 787.11 | 785.3535 | 2.406 | 2.414035 |
|  | 298.15 | 783.20 | 782.7037 | 2.055 | 2.043638 |
|  | 303.15 | 778.91 | 777.1235 | 1.791 | 1.785035 |
|  | 308.15 | 774.51 | 772.8835 | 1.560 | 1.551039 |

*Uncertainties, u, are: u(x)= ±0.0001; u(T)= ± 0.05 K; u(ρ)= ±0.15 kg.m-3; u(t)= ± 0.01s; u(η)= ± 0.001 mPa.s.

TABLE II. Densities (ρ) and viscosities (η) of the (n-heptane + alcohols) mixtures at different temperatures and atmospheric pressure

| x | ρ / kg m-3 |  |  |  |  | η / mPa s |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T / K |  |  |  |  | T / K |  |  |  |  |
|  | 288.15 | 293.15 | 298.15 | 303.15 | 308.15 | 288.15 | 293.15 | 298.15 | 303.15 | 308.15 |
| (x) n-heptane + (1-x) ethanol |  |  |  |  |  |  |  |  |  |  |
| 0.1005 | 770.87 | 764.80 | 760.84 | 756.43 | 751.70 | 1.080 | 0.985 | 0.907 | 0.819 | 0.753 |
| 0.2020 | 751.33 | 745.76 | 741.64 | 737.40 | 732.87 | 0.899 | 0.826 | 0.766 | 0.695 | 0.643 |
| 0.3022 | 736.60 | 731.34 | 727.22 | 722.84 | 718.25 | 0.782 | 0.715 | 0.667 | 0.616 | 0.569 |
| 0.4016 | 725.02 | 720.14 | 716.01 | 711.49 | 706.93 | 0.687 | 0.628 | 0.586 | 0.543 | 0.508 |
| 0.5079 | 715.08 | 710.58 | 706.41 | 701.84 | 697.37 | 0.599 | 0.567 | 0.521 | 0.486 | 0.452 |
| 0.5967 | 708.24 | 703.81 | 699.64 | 695.07 | 690.62 | 0.551 | 0.516 | 0.481 | 0.451 | 0.427 |
| 0.7041 | 701.43 | 697.06 | 692.83 | 688.47 | 684.00 | 0.500 | 0.466 | 0.442 | 0.415 | 0.396 |
| 0.8008 | 696.14 | 691.94 | 687.63 | 683.35 | 678.97 | 0.463 | 0.443 | 0.415 | 0.395 | 0.369 |
| 0.8935 | 691.89 | 687.76 | 683.40 | 679.12 | 674.67 | 0.448 | 0.423 | 0.402 | 0.383 | 0.363 |
| (x) n-heptane + (1-x) n-propanol |  |  |  |  |  |  |  |  |  |  |
| 0.1020 | 785.75 | 781.54 | 777.40 | 773.16 | 769.05 | 1.818 | 1.641 | 1.477 | 1.319 | 1.189 |
| 0.2018 | 767.57 | 763.40 | 759.30 | 754.91 | 750.70 | 1.420 | 1.291 | 1.166 | 1.047 | 0.966 |
| 0.2990 | 752.27 | 748.15 | 744.01 | 739.72 | 735.47 | 1.137 | 1.052 | 0.950 | 0.867 | 0.778 |
| 0.4005 | 738.89 | 734.60 | 730.49 | 726.12 | 721.85 | 0.914 | 0.843 | 0.769 | 0.702 | 0.644 |
| 0.5017 | 727.19 | 722.99 | 718.84 | 714.35 | 710.15 | 0.762 | 0.699 | 0.645 | 0.595 | 0.545 |
| 0.6033 | 716.75 | 712.60 | 708.40 | 704.04 | 699,76 | 0.633 | 0.590 | 0.551 | 0.506 | 0.478 |
| 0.7025 | 708.19 | 704.08 | 699.87 | 695.47 | 691.09 | 0.547 | 0.513 | 0.489 | 0.454 | 0.431 |
| 0.8010 | 700.67 | 696.47 | 692.26 | 687.94 | 683.53 | 0.499 | 0.479 | 0.442 | 0.425 | 0.399 |
| 0.8918 | 694.39 | 690.34 | 686.11 | 681.70 | 677.38 | 0.474 | 0.447 | 0.417 | 0.394 | 0.378 |
| (x) n-heptane + (1-x) iso-propanol |  |  |  |  |  |  |  |  |  |  |
| 0.1003 | 772.03 | 767.65 | 763.67 | 759.16 | 754.60 | 1.825 | 1.596 | 1.413 | 1.230 | 1.091 |
| 0.2009 | 756.15 | 752.02 | 747.94 | 743.53 | 738.83 | 1.331 | 1.178 | 1.044 | 0.927 | 0.840 |
| 0.3009 | 742.52 | 738.36 | 734.19 | 729.84 | 725.32 | 1.015 | 0.924 | 0.816 | 0.739 | 0.673 |
| 0.3980 | 731.65 | 727.47 | 723.08 | 718.63 | 714.21 | 0.825 | 0.744 | 0.685 | 0.620 | 0.577 |
| 0.5002 | 721.62 | 717.13 | 712.92 | 708.42 | 703.97 | 0.680 | 0.622 | 0.587 | 0.533 | 0.491 |
| 0.5961 | 713.33 | 708.85 | 704.65 | 700.17 | 695.74 | 0.587 | 0.548 | 0.502 | 0.468 | 0.444 |
| 0.6981 | 705.59 | 701.17 | 696.97 | 692.49 | 688.08 | 0.524 | 0.493 | 0.462 | 0.435 | 0.408 |
| 0.8014 | 698.89 | 694.59 | 690.12 | 685.66 | 681.21 | 0.484 | 0.461 | 0.427 | 0.414 | 0.380 |
| 0.9084 | 692.49 | 688.42 | 684.05 | 679.59 | 675.25 | 0.456 | 0.436 | 0.402 | 0.390 | 0.364 |

The excess molar volumes were calculated from the densities of the pure liquids and their mixtures using the following Eq. (3):

VE=xM1+1−xM2ρ−xM1ρ1+1−xM2ρ2 	(3)

where x and (1-x) are the mole fraction of the components, M1 and M2 are the molecular masses of the components 1 and 2, and ρ, ρ1 and ρ2 are the respective densities of the solution and of the pure components.

The experimental values of viscosity (η) are used to calculate the excess viscosity (ηE) defined by the Eq. (4):

ηE=η−xη1+1−xη2 	(4)

where η1 and η2 are the viscosities of pure components.

The results of VE and ηE are reported in Tables S-I and S-II of the Supplementary material. The values of excess molar volumes and excess viscosities for investigated binary mixtures in this work at 298.15 K show good agreement with literature data.4, 8

For the solution n-heptane + ethanol with x  0.4, the deviation between calculated excess molar volume value and literature value8 is 1.8%, while for excess viscosity the corresponding deviation is 0.4%. For the solution n-heptane + n-propanol with x  0.5, excess molar volume value reported in the literature4 differs from our experimental data with 2.2% and excess viscosity value differs with 0.7%.

The deviation between our excess molar volume value and literature12 value for the solution n-heptane + iso-propanol with x  0.5 is 10% and for excess viscosity is 5.0%. These differences may be due to different purity of the components used and the differences in concentrations of solutions.

Figures S-7. – S-12. of the Supplementary material to this work includes comparisons of our experimental excess molar volume and excess viscosity of n-heptane + ethanol, n-heptane + n-propanol and n-heptane + iso-propanol at 298.15 K with data available in the literature.

The excess functions of the binary systems can be represented by a Redlich-Kister type Eq. (5):

XE=xixjAk2xi−1k 	(5)

where XE represents any of the following properties: VE, ηE and xi, xj are the mole fractions of the components i and j, respectively, and k is the number of  the polynomial coefficient Ak.

The values of these coefficients are indicated in Tables III and IV along with the standard deviation, σ, defined by relation:

σ=Xexp−Xcalc2m−k1/2 	(6)

where Xexp is the experimental excess function, Xcalc is the calculated excess function with Eq. (5),  m is the number of data points and k is the number of the polynomial coefficient.

TABLE III. Coefficients Ak and standard deviations, σ, for excess molar volume at different temperatures

| T / K | 106×A0 / m3 mol-1 | 106×A1 / m3 mol-1 | 106×A2 / m3 mol-1 | 106×A3 / m3 mol-1 | 106× σ / m3 mol-1 |
| --- | --- | --- | --- | --- | --- |
| (x) n-heptane + (1-x) ethanol |  |  |  |  |  |
| 288.15 | 1.725 | -0.044 | 0.310 | 1.241 | 0.008 |
| 293.15 | 1.764 | -0.111 | 0.585 | 1.127 | 0.006 |
| 298.15 | 1.791 | -0.176 | 0.836 | 1.517 | 0.007 |
| 303.15 | 1.974 | -0.088 | 0.553 | 1.137 | 0.013 |
| 308.15 | 2.045 | -0.111 | 0.691 | 1.270 | 0.019 |
| (x) n-heptane + (1-x) n-propanol |  |  |  |  |  |
| 288.15 | 0.920 | 0.499 | 0.421 | -0.082 | 0.015 |
| 293.15 | 1.031 | 0.528 | 0.520 | -0.109 | 0.012 |
| 298.15 | 1.087 | 0.548 | 0.756 | -0.387 | 0.015 |
| 303.15 | 1.198 | 0.526 | 0.879 | -0.370 | 0.016 |
| 308.15 | 1.265 | 0.551 | 1.141 | -0.476 | 0.012 |
| (x) n-heptane + (1-x) iso-propanol |  |  |  |  |  |
| 288.15 | 1.172 | -0.107 | 0.492 | 0.818 | 0.016 |
| 293.15 | 1.398 | 0.458 | 0.469 | -0.177 | 0.018 |
| 298.15 | 1.480 | 0.306 | 0.743 | 0.659 | 0.015 |
| 303.15 | 1.581 | 0.451 | 0.951 | 0.566 | 0.020 |
| 308.15 | 1.653 | 0.479 | 1.225 | 0.372 | 0.016 |

TABLE IV. Coefficients Ak and standard deviations, σ, for excess viscosities at different temperatures

| T / K | A0 / mPa s | A1 / mPa s | A2 / mPa s | A3 / mPa s | σ / mPa s |
| --- | --- | --- | --- | --- | --- |
| (x) n-heptane + (1-x) ethanol |  |  |  |  |  |
| 288.15 | -1.171 | 0.442 | -0.567 | 0.440 | 0.004 |
| 293.15 | -1.010 | 0.418 | -0.426 | 0.185 | 0.003 |
| 298.15 | -0.895 | 0.340 | -0.270 | 0.174 | 0.002 |
| 303.15 | -0.796 | 0.274 | -0.306 | 0.292 | 0.003 |
| 308.15 | -0.715 | 0.271 | -0.292 | 0.201 | 0.003 |
| (x) n-heptane + (1-x) n-propanol |  |  |  |  |  |
| 288.15 | -2.825 | 1.226 | -0.992 | 1.094 | 0.006 |
| 293.15 | -2.482 | 1.003 | -0.796 | 1.085 | 0.002 |
| 298.15 | -2.164 | 0.972 | -0.702 | 0.681 | 0.006 |
| 303.15 | -1.892 | 0.7853 | -0.590 | 0.725 | 0.002 |
| 308.15 | -1.626 | 0.718 | -0.309 | 0.307 | 0.006 |
| (x) n-heptane + (1-x) iso-propanol |  |  |  |  |  |
| 288.15 | -3.657 | 2.288 | -2.053 | 1.631 | 0.010 |
| 293.15 | -3.121 | 1.919 | -1.756 | 1.590 | 0.012 |
| 298.15 | -2.563 | 1.639 | -1.399 | 0.907 | 0.005 |
| 303.15 | -2.196 | 1.367 | -1.170 | 1.016 | 0.006 |
| 308.15 | -1.841 | 1.119 | -1.019 | 0.752 | 0.006 |

Several effects may contribute to the values of excess molar volumes, such as breaking of liquid order on mixing, unfavorable interaction between groups, differences in molecular volumes and differences in free volumes between liquids components.40

Fig. 1 shows experimental (Eq. 3) and calculated values (Eq. 5) of VEat 298.15 K constant temperature.

Fig. 1. Experimental excess molar volumes versus mole fraction x at 298.15 K for the mixtures:

(■) (x) n-heptane + ethanol; (▲) (x) n-heptane + n-propanol; () (x) n-heptane + iso-propanol.

Continuous curve was calculated from Eq. (5) for experimental data.

The experimental excess molar volumes for all the binary mixtures studied in this work are positive in the whole composition range and at all temperatures. The positive values are due to expansion of solution volume due to mixing caused by the hydrogen bond rupture and dispersive interactions between unlike molecules. In this sense, the addition of non-polar solvent causes disruption of alcohol aggregates through the breaking of hydrogen bonds, making VE positive, since aggregates have higher volumes than the sum of their components.41

The curves for n-heptane + n-propanol and n-heptane + iso-propanol systems are asymmetric, with its maximum displace toward a high mole fraction of n-heptane; this behavior agrees with the literature.42-44 The excess molar volumes decreases with increasing alkyl chain length of the alcohol. The excess molar volumes of iso-propanol + n-heptane solutions is greater than excess molar volumes of n-propanol + n-heptane mixtures. These behavior is in accord with Ortega et al.45 that showed that VE values are greater when alkanes are mixed with secondary or tertiary alcohols than when mixed with primary alcohols. Breaking the hydrogen bonds by adding a non-polar solvent is more pronounced for iso-propanol compared with n-propanol. This may explain the higher values of excess molar volumes in iso-propanol solutions than in n-propanol solutions. The effect of temperature on the VE shows a systematic decrease with temperature for all the mixtures.

The excess viscosities values may be generally explained by considering the difference in size and shape of the component molecules and specific interactions between unlike molecules such as H-bond formation and charge transfer complexes may cause increase in viscosity in mixtures rather than in pure component.46 Positive values of excess viscosities are indicate of strong interactions, while negative values indicate weaker interactions.47

Figure 2 shows experimental (Eq. 4) and calculated values (Eq. 5) of ηE at 303.15 K constant temperature.

Fig. 2. Experimental excess viscosities versus mole fraction x at 303.15 K for the mixtures:

(■) (x) n-heptane + ethanol; (▲) (x) n-heptane + n-propanol; ()  (x) n-heptane + iso-propanol.

Continuous curve was calculated from Eq. (5) for experimental data.

The excess viscosities are negative over the entire range of mole fractions at all the temperatures. The negative excess viscosity was explained by many authors through different forms.48-51 The negative values support the main factor of breaking of the self-associated alcohols and weak interactions between unlike molecules.52 The minima of ηE-x curves occur at low mole fraction of n-heptane. This behavior is characteristic of mixtures of a substance that exhibits molecular association and another that does not.4 If the temperature increases the negative values of excess viscosity decrease. This can be explained by breaking hydrogen bonds and increasing mobility of molecules due of the increase of thermal energy.33

Flory and Prigogine-Flory-Patterson theories

Experimental results of the molar excess volume have been used to test the applicability of the Flory13-16 and Prigogine-Flory-Patterson (PFP) theories.17, 18

In the Flory theory, the excess molar volume is related to the reduced volumev, the reduced temperature Tand the adjustable interactional parameterχ12. The PFP theory includes three contributions in order to explain the thermodynamic behavior of the liquid mixtures: an interactional contribution VintE, which is proportional to the interactional parameter χ12, the free volume contribution VFVE, which arises from the dependence of the reduced volume upon the reduced temperature as a result of the difference between the degree of expansion of the two components and the internal pressure contribution VpE, which depends on both the difference of characteristic pressures and the reduced volumes of the components.

The excess molar volume given by the Flory theory is:

VE=x1V1+x2V2ϕ1ν1+ϕ2ν273T−T043−ϕ1ν1+ϕ2ν213 	(7)

The Prigogine Flory Patterson Equation is:

VEx1V1+x2V2=ν13−1ν2343ν−13−1ψ1θ2χ12p1−ν1−ν22149ν−13−143ν−13−1νψ1ψ2+ν1−ν2p1−p2ψ1p2+ψ2p1ψ1ψ2=VintEx1V1+x2V2+VFVEx1V1+x2V2+VpEx1V1+x2V2 	(8)

where:

vis the reduced volume:

ν=αT31+αT+13 	(9)

V- characteristic volume;

p∗- characteristic pressure;

T- reduced temperature:

T=ϕ1p1T1+ϕ2p2T2ϕ1p1+ϕ2p2−ϕ1θ2χ12 	(10)

T0- ,,ideal” reduced temperature for the mixture;

ψi- contact energy fraction:

ψ1=1−ψ2=ϕ1p1/ϕ1p1+ϕ2p2 		(11)

ϕi- hard core volume fraction:

ϕ2=1−ϕ1=x2V2/x1V1+x2V2                                                (12)

θi- surface site fraction:

θ2=1−θ1=ϕ2V213/ϕ1V113+ϕ2V213                            (13)

The coefficient of thermal expansion, α, was obtained from the following equation: 53

α=1/V∂V/∂Tp,                                                         (14)

where:

V  is the molar volume of the solution;

T - temperature.

The various parameters involved in Eqs. 7 and 8 for the pure components and the mixture are obtained from Flory’s theory13, 14 and are shown in Tables V and VI.

The values of the interaction parameter 12, were derived by fitting the theory to experimental values of the excess molar volume for equimolar solutions. The results of VE calculated from Eqs. 7 and 8 are reported in Table VII. These results were compared by means of the percentage absolute average deviation (ADD) with the following equation:

ADD%=1ni=1nQexp−QcalcQexp.100 	(15)

where n is the number of experimental data and Q represents VE.

TABLE V. Parameters of the pure components, coefficient of thermal expansion α, isothermal compressibility kT, reduced volume v, characteristic volume V*, characteristic pressure P*, characteristic temperature T*, and reduced temperature T for Flory theory

| Component | 103.α / K-1 |  | kT / 104.MPa-1 | v | 106.V* / m3 mol-1 | 106.p*/ J m-3 | T*/ K | T |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 293.15 K |  |  |  |  |  |  |  |  |
| n-Heptane |  | 1.24 | 14.3854 | 1.2909 | 113.4584 | 421.0756 | 4638.1 | 0.0632 |
| Ethanol |  | 1.19 | 11.0555 | 1.2815 | 45.5330 | 518.5073 | 4733.6 | 0.0619 |
| n-Propanol |  | 0.97 | 9.5556 | 1.2381 | 60.3438 | 456.4552 | 5281.1 | 0.0555 |
| iso-Propanol |  | 1.07 | 10.8156 | 1.2583 | 60.6829 | 459.4142 | 5003.3 | 0.0586 |
| 298.15 K |  |  |  |  |  |  |  |  |
| n-Heptane |  | 1.24 | 14.60657 | 1.2948 | 113.8119 | 424.2082 | 4678.1 | 0.0637 |
| Ethanol |  | 1.19 | 11.5358 | 1.2854 | 45.6163 | 508.4346 | 4773.5 | 0.0624 |
| n-Propanol |  | 0.97 | 10.2659 | 1.2415 | 60.4575 | 434.4775 | 5320.4 | 0.0560 |
| iso-Propanol |  | 1.07 | 11.3058 | 1.2619 | 60.8110 | 449.5515 | 5042.9 | 0.0591 |
| 303.15 K |  |  |  |  |  |  |  |  |
| n-Heptane |  | 1.24 | 16.355 | 1.2987 | 114.1912 | 388.835 | 4718.2 | 0.0642 |
| Ethanol |  | 1.19 | 11.9555 | 1.2892 | 45.7197 | 501.7603 | 4813.5 | 0.0630 |
| n-Propanol |  | 0.97 | 10.5756 | 1.2449 | 60.5970 | 431.1326 | 5359.7 | 0.0566 |
| iso-Propanol |  | 1.07 | 11.8260 | 1.2655 | 60.9737 | 439.4645 | 5082.5 | 0.0596 |
| 308.15 K |  |  |  |  |  |  |  |  |
| n-Heptane |  | 1.24 | 17.761 | 1.3026 | 114.5799 | 366.1661 | 4758.4 | 0.0647 |
| Ethanol |  | 1.19 | 12.362 | 1.2930 | 45.8555 | 498.4433 | 4853.5 | 0.0635 |
| n-Propanol |  | 0.97 | 11.0833 | 1.2482 | 60.7244 | 420.3194 | 5399.0 | 0.0571 |
| iso-Propanol |  | 1.07 | 12.3233 | 1.2690 | 61.1482 | 430.9963 | 5122.2 | 0.0602 |

TABLE VI. Parameters of the liquid mixtures for Flory and Prigogine-Flory-Patterson theories

| T / K |  |  | v | T | 12.106 / J m-3 | 1 |
| --- | --- | --- | --- | --- | --- | --- |
| (x) n-heptane + (1-x) ethanol |  |  |  |  |  |  |
| 293.15 | 0.2864 | 0.2284 | 1.2938 | 0.0636 | 34.9794 | 0.6692 |
| 298.15 | 0.2861 | 0.2281 | 1.2977 | 0.0641 | 34.3676 | 0.6755 |
| 303.15 | 0.2859 | 0.2278 | 1.3022 | 0.0647 | 35.3985 | 0.6593 |
| 308.15 | 0.2858 | 0.2277 | 1.3062 | 0.0652 | 34.9129 | 0.6473 |
| (x) n-heptane + (1-x) n-propanol |  |  |  |  |  |  |
| 293.15 | 0.3472 | 0.3012 | 1.2755 | 0.0611 | 25.2022 | 0.6343 |
| 298.15 | 0.3469 | 0.3008 | 1.2794 | 0.0616 | 21.7395 | 0.6476 |
| 303.15 | 0.3467 | 0.3005 | 1.2835 | 0.0622 | 26.2681 | 0.6296 |
| 308.15 | 0.3464 | 0.3001 | 1.2874 | 0.0627 | 27.4530 | 0.6217 |
| (x) n-heptane + (1-x) iso-propanol |  |  |  |  |  |  |
| 293.15 | 0.3485 | 0.3027 | 1.2835 | 0.0622 | 24.7486 | 0.6315 |
| 298.15 | 0.3482 | 0.3024 | 1.2876 | 0.0627 | 24.2387 | 0.6385 |
| 303.15 | 0.3481 | 0.3022 | 1.2917 | 0.0633 | 25.8128 | 0.6236 |
| 308.15 | 0.3480 | 0.3021 | 1.2956 | 0.0638 | 26.4277 | 0.6142 |

TABLE VII. Experimental and calculated excess volumes and calculated values of the three contributions to VE at 0.5 mole fraction

| T / K | 106.VEexp. / m3 mol-1 | 106.VEFlory/ m3 mol-1 | 106.VEPFP / m3 mol-1 | 106.VEint. / m3 mol-1 | 106.VEFV / m3 mol-1 | 106.VEp* / m3 mol-1 | ADD% Flory | ADD% PFP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (x) n-heptane + (1-x) ethanol |  |  |  |  |  |  |  |  |
| 293.15 298.15 303.15 308.15 | 0.441 0.448 0.493 0.511 | 0.437 0.444 0.489 0.506 | 0.445 0.452 0.499 0.517 | 0.480 0.483 0.543 0.571 | 0.002 0.002 0.002 0.002 | -0.033 -0.029 -0.042 -0.052 | 0.9 0.9 0.8 0.9 | 0.9 0.9 1.2 1.0 |
| (x) n-heptane + (1-x) n-propanol |  |  |  |  |  |  |  |  |
| 293.15 298.15 | 0.258 0.272 | 0.257 0.270 | 0.262 0.276 | 0.430 0.385 | 0.083 0.084 | -0.085 -0.025 | 0.4 0.7 | 1.5 1.5 |
| 303.15 308.15 | 0.299 0.316 | 0.298 0.314 | 0.305 0.323 | 0.504 0.565 | 0.088 0.090 | -0.111 -0.152 | 0.3 0.6 | 2.0 2.2 |
| (x) n-heptane + (1-x) iso-propanol |  |  |  |  |  |  |  |  |
| 293.15 298.15 303.15 308.15 | 0.349 0.370 0.395 0.413 | 0.347 0.368 0.393 0.410 | 0.353 0.374 0.400 0.419 | 0.442 0.444 0.516 0.564 | 0.032 0.032 0.034 0.034 | -0.057 -0.038 -0.082 -0.111 | 0.6 0.5 0.5 0.7 | 1.1 1.1 1.3 1.4 |

The results in Table VII reveal that ADD values in the predictions of excess molar volume for

n-heptane + ethanol mixtures are less than 0.9% using the Flory theory and less than 1.2% using Prigogine-Flory-Patterson theory. For n-heptane + n-propanol system the ADD value is 0.7% using the Flory theory and less than 2.2 % using Prigogine-Flory-Patterson theory. For n-heptane + iso-propanol system the ADD values are less than 0.7% for Flory theory and less than 1.4 % using Prigogine-Flory-Patterson theory. These low values of percentage absolute average deviation obtained suggest the validity of these theories. An analysis of each of the three contributions to excess molar volume shows that the interaction contribution is positive, being the most important contribution. The volume contribution is positive, while the internal pressure contribution is negative.

The viscosity data correlation

Several empirical and semi-empirical relations have been used to represent the dependence of viscosity on concentration of components in binary systems. These relations are classified according to the number of adjustable parameters used to account for the deviation from some average.63, 64

Grunberg-Nissan65 proposed the following equation based on one parameter:

lnη=x1lnη1+x2lnη2+x1x2d 	(16)

Heric-Brewer66 expression with two parameters is:

lnη=x1lnη1+x2lnη2+x1lnM1+x2lnM2−lnx1M1+x2M2+x1x2α12+α21x1−x2 (17)

The three body McAllister67 Equation with two parameters is:

lnη=x13lnη1+3x12x2lnη12+3x1x22lnη21+x23lnη2−lnx1+x2M2M1+3x12x2ln2+M2M13+3x1x22ln1+2M2M13+x23lnM2M1 	(18)

We tested also the four body McAllister Equation with three parameters:

lnη=x14lnη1+4x13x2lnη1112+6x12x22lnη1122+4x1x23lnη2221+x24lnη2−lnx1+x2M2M1+4x13x2ln3+M2M14+6x12x22ln1+M2M12+4x1x23ln1+3M2M14+x24lnM2M1 	(19)

and Jouyban Acree68,69 Equation with four parameters:

lnη=x1lnη1+x2lnη2+x1x2Ajx1−x2jT, where n=3 	(20)

In the Eqs. 16-20 the dynamic viscosity of the liquid mixture is designated by η and η1, η2 are the dynamic viscosities, x1, x2 are the mole fractions of the pure components 1 and 2 constituting the liquid mixture, M1, M2 are the molecular masses, T is the temperature; d, α12, , Aj are interaction parameters (viscosity coefficients) and reflect the non-ideality of the system and the McAllister model adjustable parameters are given by η12, η21, η1112, η1122 and η2221. The parameters that appear in Eqs. 16-20 were estimated using the experimental viscosity data and a non-linear regression analysis employing the Levenberg-Marquardt algorithm.70

Table VIII shows the parameters calculated, the standard deviations (σ) and the percentage absolute average deviation (ADD) values between experimental values and those obtained using the semi-empirical relations. The standard deviation was calculated using the Eq. 6, were where Xexp is the experimental viscosity, Xcalc is the calculated viscosity with Eqs. 16-20 and k is the number of adjustable parameters. The ADD values were determined for all equations with the Eq. 15, were Q represents the values of viscosity.

The interactional parameter d is negative for binary systems. Nigam and Mahl71 concluded from the study of binary mixtures that if ηE< 0, d < 0 and magnitude of both are large then the dispersion force would be dominant.  Applying the model with one parameter (Grunberg-Nissan model) gives the highest values of standard deviation. Models with two parameters (Heric-Brewer and three body McAllister model) lead to identical result of standard deviation and their application leads to a poor estimate of viscosities solutions. These models are not suitable to estimate viscosities of the systems studied in this work. Use of three and four parameter equations reduces the σ values. The data show that the Jouyban-Acree model is suitable for 288.15 K, 298.15 K and 303.15 K, while the four body McAllister model is adequate at 293.15 K and 308.15 K for n-heptane and ethanol system. The Jouyban-Acree model shows the best agreement with experimental data for n-heptane + n-propanol system at all temperatures. A comparison of the calculated and the experimental viscosities for the n-heptane + iso-propanol mixtures shows that the Jouyban-Acree model produces accurate results for 288.15 K, 293.15 K, 303.15 K and 308.15 K, while for 298.15 K the four body McAllister model is able to describe viscosities. From this study, it can be concluded that the correlating ability significantly improves for these non-ideal systems as number of adjustable parameters is increased.

TABLE VIII. Parameters for the semi-empirical relations of Grunberg-Nissan, Heric-Brewer, Joyban-Acree and McAllister, σ and ADD at different temperatures

| Equation | Parameters and σ | T / K |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 288.15 | 293.15 | 298.15 | 303.15 | 308.15 |
| (x) n-heptane + (1-x) ethanol |  |  |  |  |  |  |
| Grunberg-Nissan | d σ/ADD% | -1.05252 0.014/7.6 | -0.99463 0.011/7.0 | -0.9423 0.008/2.3 | -0.95219 0.011/2.4 | -0.93666 0.014/2.1 |
| Heric-Brewer | σ/ADD% | -0.73862 -0.02886 0.014/7.1 | -0.68155 -0.02088 0.011/6.9 | -0.62953 -0.03112 0.005/1.5 | -0.64026 0.00233 0.008/1.8 | -0.62536 0.01903 0.011/1.9 |
| McAllister three body model | η12 η21 σ/ADD% | 0.46989 0.6893 0.014/6.7 | 0.44581 0.63852 0.011/6.6 | 0.41949 0.59849 0.005/1.5 | 0.39799 0.54503 0.008/1.7 | 0.37744 0.50516 0.011/1.8 |
| McAllister four body model | η1112 η1122 η2221 σ/ADD% | 0.42229 0.66138 0.74053 0.006/0.9 | 0.40837 0.59839 0.69382 0.005/0.7 | 0.39952 0.52336 0.67174 0.004/0.6 | 0.37353 0.50186 0.60047 0.005/0.7 | 0.35066 0.47945 0.54911 0.007/0.9 |
| Jouyban-Acree | A0 A1 A2 A3 σ/ADD% | -280.5709 -6.63531 -125.80538 62.8769 0.005/0.8 | -272.64576 15.19357 -100.461 4.39366 0.006/1.0 | -269.73936 2.65942 -46.58031 32.89759 0.003/0.2 | -273.94864 -5.06481 -72.3092 87.8755 0.004/0.5 | -271.72153 17.68567 -88.53998 36.24159 0.008/1.1 |

TABLE VIII. continued

| Equation | Parameters and σ | T / K |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 288.15 | 293.15 | 298.15 | 303.15 | 308.15 |
| (x) n-heptane + (1-x) n-propanol |  |  |  |  |  |  |
| Grunberg-Nissan | d σ/ADD% | -1.33679 0.013/4.2 | -1.2972 0.013/4.4 | -1.2595 0.007/3.3 | -1.26948 0.011/4.1 | -1.20592 0.008/3.8 |
| Heric-Brewer | σ/ADD% | -1.20598 -0.08898 0.012/3.3 | -1.16647 -0.06224 0.013/4.4 | -1.12892 -0.01501 0.007/3.3 | -1.1389 -0.01511 0.011/4.0 | -1.07537 -0.00534 0.008/3.8 |
| McAllister three body model | η12 η21 σ/ADD% | 0.49779 0.93537 0.012/3.3 | 0.4726 0.8565 0.013/4.4 | 0.44832 0.77409 0.007/3.3 | 0.41763 0.70264 0.011/4.0 | 0.39635 0.64614 0.008/3.8 |
| McAllister four body model | η1112 η1122 η2221 σ/ADD% | 0.46839 0.7177 1.15582 0.012/2.6 | 0.45617 0.63813 1.08093 0.013/4.4 | 0.42547 0.606 0.95981 0.006/3.0 | 0.40388 0.54586 0.87642 0.011/4.0 | 0.40052 0.47138 0.83345 0.006/3.2 |
| Jouyban-Acree | A0 A1 A2 A3 σ/ADD% | -379.11066 -88.7144 -41.73164 210.91809 0.006/2.0 | -379.399 -93.79076 -5.99564 248.4637 0.006/2.0 | -371.80202 -37.18418 -25.78288 119.47068 0.004/1.7 | -383.44331 -54.849 -9.84334 172.7069 0.008/2.2 | -379.39411 -19.19074 53.53771 73.10081 0.005/1.9 |
| (x) n-heptane + (1-x) iso-propanol |  |  |  |  |  |  |
| Grunberg-Nissan | d σ/ADD% | -1.97244 0.031/12.0 | -1.9136 0.038/15.0 | -1.79896 0.035/13.0 | -1.76861 0.042/16.5 | -1.68819 0.035/13.1 |
| Heric-Brewer | σ/ADD% | -1.83979 0.41595 0.009/2.6 | -1.78048 0.51663 0.010/3.5 | -1.66612 0.45655 0.012/4.1 | -1.6352 0.57744 0.010/3.1 | -1.55532 0.46338 0.011/3.8 |
| McAllister three body model | η12 η21 σ/ADD% | 0.49333 0.68538 0.009/2.6 | 0.47907 0.60539 0.010/3.5 | 0.44349 0.5654 0.012/4.1 | 0.43343 0.49375 0.010/3.1 | 0.39538 0.47218 0.011/3.8 |
| McAllister four body model | η1112 η1122 η2221 σ/ADD% | 0.45835 0.62695 0.92639 0.007/2.2 | 0.45023 0.56269 0.82996 0.010/3.4 | 0.40464 0.55432 0.7338 0.008/1.5 | 0.40854 0.47969 0.66445 0.010/3.0 | 0.36054 0.48334 0.59487 0.007/2.0 |
| Jouyban-Acree | A0 A1 A2 A3 σ/ADD% | -559.21197 88.41773 -60.96694 113.78884 0.004/0.8 | -554.91678 102.37527 -37.97912 167.91459 0.005/0.9 | -523.96268 128.81444 -83.47631 41.00496 0.009/1.6 | -530.666 133.33436 -33.11152 146.37661 0.007/1.3 | -506.19214 126.73246 -94.84874 68.01922 0.006/1.4 |

Thermodynamic functions of activation

The energies of activation of viscous flow for the binaries studied were calculated using the following Eq. (21): 72

η=hNVexpΔGRT 	(21)

where η is viscosity of a liquid mixtures, h is Planck`s constant, N  is Avogadro`s number, V is the molar volume of the solution, R is general gas constant, T is temperature and G is the molar Gibbs energy of activation for the viscous flow process. Combining with

ΔG=ΔH−TΔS                                                          (22)

yields the equation

lnηVhN=ΔHRT−ΔSR 	(23)

where ΔHand ΔS are enthalpy and entropy of activation of viscous flow.

The plots of lnηVhN vs 1/T were found to be linear in the temperature range 288.15 to 308.15 K, as such the values of ΔH and ΔSwere obtained by the corresponding slopes and the intercepts. With ΔH and ΔS values as input in Eq. 22, the corresponding values of ΔG were also calculated. The values of thermodynamic functions of activation of viscous flow and of correlation coefficient (r) are listed in Table IX as a function of composition. The experimental values of ΔG against mole fraction are presented in Figs. S-13 - S-15 of the Supplementary material.

TABLE IX. Values of ΔG, ΔH, ΔSand r for the binary mixtures

| x | ΔG / J mol-1 |  |  |  |  | ΔH / J mol-1 | ΔS/ J mol-1K-1 |  | r |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T / K |  |  |  |  |  |  |  |  |
|  | 288.15 | 293.15 | 298.15 | 303.15 | 308.15 |  |  |  |  |
| (x) n-heptane + (1-x) ethanol |  |  |  |  |  |  |  |  |  |
| 0.0000 | 12660.30 | 12644.09 | 12627.88 | 12611.67 | 12595.46 | 13594.50 | 3.24 | 0.9992 |  |
| 0.1005 | 12461.31 | 12461.22 | 12461.13 | 12461.05 | 12460.96 | 12466.34 | 0.02 | 0.9978 |  |
| 0.2020 | 12326.33 | 12340.18 | 12354.04 | 12367.89 | 12381.74 | 11528.04 | -2.77 | 0.9971 |  |
| 0.3022 | 12245.13 | 12272.84 | 12300.54 | 12328.25 | 12355.95 | 10648.58 | -5.54 | 0.9984 |  |
| 0.4016 | 12162.76 | 12197.21 | 12231.67 | 12266.12 | 12300.57 | 10177.15 | -6.89 | 0.9983 |  |
| 0.5079 | 12094.13 | 12136.69 | 12179.26 | 12221.82 | 12264.39 | 9641.12 | -8.51 | 0.9933 |  |
| 0.5967 | 12047.06 | 12106.89 | 12166.72 | 12226.54 | 12286.37 | 8599.30 | -11.96 | 0.9984 |  |
| 0.7041 | 12000.25 | 12074.95 | 12149.66 | 12224.36 | 12299.07 | 7694.94 | -14.94 | 0.9960 |  |
| 0.8008 | 12004.89 | 12084.22 | 12163.54 | 12242.87 | 12322.19 | 7433.39 | -15.86 | 0.9890 |  |
| 0.8935 | 12053.60 | 12145.30 | 12236.99 | 12328.69 | 12420.39 | 6769.07 | -18.34 | 0.9992 |  |
| 1.0000 | 12178.33 | 12269.74 | 12361.14 | 12452.55 | 12543.95 | 6910.59 | -18.28 | 0.9948 |  |
| (x) n-heptane + (1-x) n-propanol |  |  |  |  |  |  |  |  |  |
| 0.0000 | 14730.52 | 14696.83 | 14663.13 | 14.629.43 | 14595.74 | 16672.46 | 6.74 | 0.9970 |  |
| 0.1020 | 14194.50 | 14181.02 | 14167.54 | 14154.06 | 14140.58 | 14971.27 | 2.69 | 0.9986 |  |

TABLE IX. continued

| x | ΔG / J mol-1 |  |  |  |  | ΔH / J mol-1 | ΔS/ J mol-1K-1 | r |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T / K |  |  |  |  |  |  |  |  |  |  |
|  | 288.15 | 293.15 | 298.15 | 303.15 | 308.15 |  |  |  |  |  |  |
| (x) n-heptane + (1-x) n-propanol |  |  |  |  |  |  |  |  |  |  |  |
| 0.2018 | 13795.95 | 13798.36 | 13800.77 | 13803.18 | 13805.59 | 13657.12 | -0.48 | 0.9979 |  |  |  |
| 0.4005 | 13103.01 | 13119.76 | 13136.51 | 13153.26 | 13170.01 | 12137.60 | -3.35 | 0.9985 |  |  |  |
| 0.5017 | 12820.76 | 12845.94 | 12871.12 | 12896.30 | 12921.47 | 11369.77 | -5.03 | 0.9990 |  |  |  |
| 0.6033 | 12531.08 | 12580.81 | 12630.53 | 12680.26 | 12729.99 | 9665.25 | -9.94 | 0.9954 |  |  |  |
| 0.7025 | 12318.84 | 12394.59 | 12470.33 | 12546.08 | 12621.82 | 7953.70 | -15.15 | 0.9946 |  |  |  |
| 0.8010 | 12242.40 | 12324.62 | 12406.83 | 12489.05 | 12571.27 | 7504.20 | -16.44 | 0.9813 |  |  |  |
| 0.8918 | 12214.68 | 12294.00 | 12373.33 | 12452.65 | 12531.97 | 7643.22 | -15.86 | 0.9917 |  |  |  |
| 1.0000 | 12178.33 | 12269.74 | 12361.14 | 12452.55 | 12543.95 | 6910.59 | -18.28 | 0.9948 |  |  |  |
| (x) n-heptane + (1-x) iso-propanol |  |  |  |  |  |  |  |  |  |  |  |
| 0.0000 | 15018.63 | 14924.62 | 14830.61 | 14736.60 | 14642.59 | 20436.51 | 18.80 |  | 0.9993 |  |  |
| 0.1003 | 14234.64 | 14166.02 | 14097.40 | 14028.79 | 13960.17 | 18189.04 | 13.72 |  | 0.9993 |  |  |
| 0.2009 | 13664.96 | 13619.54 | 13574.12 | 13528.70 | 13483.28 | 16282.43 | 9.08 |  | 0.9990 |  |  |
| 0.3009 | 13210.61 | 13186.93 | 13163.25 | 13139.56 | 13115.88 | 14575.30 | 4.74 |  | 0.9971 |  |  |
| 0.3980 | 12854.01 | 12862.86 | 12871.70 | 12880.54 | 12889.39 | 12344.36 | -1.77 |  | 0.9970 |  |  |
| 0.5002 | 12566.97 | 12595.02 | 12623.07 | 12651.12 | 12679.16 | 10950.53 | -5.61 |  | 0.9923 |  |  |
| 0.5961 | 12347.29 | 12393.98 | 12440.66 | 12487.34 | 12534.02 | 9656.94 | -9.34 |  | 0.9927 |  |  |
| 0.6981 | 12223.10 | 12291.17 | 12359.24 | 12427.31 | 12495.38 | 8300.32 | -13.61 |  | 0.9994 |  |  |
| 0.8014 | 12172.33 | 12248.78 | 12325.23 | 12401.67 | 12478.12 | 7766.80 | -15.29 |  | 0.9735 |  |  |
| 0.9084 | 12156.33 | 12239.05 | 12321.77 | 12404.48 | 12487.20 | 7389.39 | -16.54 |  | 0.9784 |  |  |
| 1.0000 | 12178.33 | 12269.74 | 12361.14 | 12452.55 | 12543.95 | 6910.59 | -18.28 |  | 0.9948 |  |  |

The values of ΔH are more than two times higher for ethanol and n-propanol than for n-heptane, while for iso-propanol the values are more than three times higher than that for n-heptane, indicating that association and dipole-dipole interactions increase the value of  ΔH.

The value of S is negative for n-heptane and positive for ethanol, n-propanol and iso-propanol and shows that overall molecular order due to activated complex formation increases for n-heptane (nonassociating component) but decreases in case of ethanol, n-propanol and iso-propanol. The positive S for all alcohols indicated that, probably rupturing of hydrogen bonds formed through OH groups of alcohols in the activation process for viscous flow occurs, resulting in the structural disorder.73 The values of H and G are positive for all the binary mixtures and at all temperatures used. These values of free energy and enthalpy of activation of viscous flow increase with the alcohol concentration of solution at temperature constant. The values of G at constant concentration decrease if the temperature increase in accord with viscosity variation except for the mixtures concentrated in alcohols. The values of S are negative for all binary mixtures except for the mixtures concentrated in alcohols. These values show that overall molecular order due to activated complex formation increases for all binary mixtures except for the mixtures concentrated in alcohols.74

The excess molar Gibbs energies of activation of viscous flow, GE, were obtained from following Eq. (24):

ΔGE=RTlnηV−xlnη1V1+1−xlnη2V2	(24)

where V is the molar volume of the mixture and V1 and V2 are the respective molar volumes of the pure components. The ΔGEparameter may be considerable a valid measure to detect the presence of interaction between molecules.75, 76 Negative values were observed for excess molar Gibbs energies of activation of viscous flow of the all the binary mixtures and at all temperatures studied. These values lead us to conclude that dispersion forces are the predominant in these systems. The experimental values of ΔGE against mole fraction are presented in Figs. S-16 - S-18 of the Supplementary material.

CONCLUSIONS

The densities and viscosities of binary mixtures of n-heptane with alcohols (ethanol, n-propanol and iso-propanol) were measured experimentally at temperatures between 288.15 and 308.15 K over the entire composition range. From these results, the excess values of molar volume and viscosity have been computed and fitted to the Redlich-Kister Equation. Positive deviations were observed for excess molar volumes and negative deviations for the excess dynamic viscosity at all of the temperatures and in the whole concentration range. Experimental results of the molar excess volume have been used to test the applicability of the Flory and Prigogine-Flory-Patterson theories. The values of percentage absolute average deviation obtained suggest the validity of these theories. Grunberg-Nissan, Heric-Brewer, Jouyban Acree, three-body and four-body McAlister models have been used to calculate viscosity coefficients and these were compared with experimental data for the mixtures. The results of these correlations showed that Jouyban Acree and four-body McAllister models are suitable to describe viscosities of the binary mixtures studied in this work.

The energies of activation of viscous flow and excess molar Gibbs energies of activation of viscous for these binary mixtures were also calculated. The values of ΔH and ΔG are positive for all the binary mixtures at all of the temperatures and in the whole concentration range. The values of ΔS are negative for all binary mixtures except for the mixtures concentrated in alcohols. Negative deviations were observed for ΔGE of the all systems and at all of the temperatures.

## REFERENCES

1. D. Gomez-Diz, J.C. Mejuto, J.M. Navaza, A. Rodriguez-Alvarez, J. Chem. Eng. Data 47 (2002) 872

2. J.A. Estrada-Baltazar, F.J. Juan, A. Gustavo,  J. Chem. Eng. Data 43 (1998) 441

3. H. Iloukhani, M. Jafari, Monatsh. Chem. 142 (2011) 1005

4. E. Jimenez, C. Franjo, L. Segade, J.L. Legido, M.I. Paz Andrade, Journal of Solution Chemistry 27 (1998) 569

5.  M. Keller, S. Schnabel, A.  Heintz, Fluid Phase Equilib. 110 (1995) 231

6. R. Tanaka, S. Toyama, J. Chem. Thermodynamics 28 (1996) 1403

7. A.B. Pereiro, A. Rodriguez, J. Chem. Thermodynamics 39 (2007) 1219

8. B. Orge, M. Iglesias, A. Rodriguez, J.M. Canosa, J. Tojo, Fluid Phase Equilib. 133 (1997) 213

9. D. Papaioannou, D. Ziakas, C. Panayiotou, J. Chem. Eng. Data 36 (1991) 35

10. C.K. Zeberg-Mikkelsen, G. Watson, A. Baylaucq, G. Galliero, C. Boned, Fluid Phase Equilib. 245 (2006) 6

11. J. Zielkiewicz, J. Chem. Thermodynamics 26 (1994) 959

12. V. Rajendran, Acta Ciencia Indica Phys. 21 (1995) 69

13. P.J. Flory, J. Am. Chem. Soc. 87 (1965) 1833

14. A. Abe, P.J. Flory, J. Am. Chem. Soc. 87 (1965) 1838

15. P.J. Flory, R.A. Orwoll, A. Vrij, J. Am. Chem. Soc. 86 (1964) 3507

16. R.A. Orwoll, P.J. Flory, J. Am. Chem. Soc. 89 (1967) 6814

17. H.T. Van, D. Patterson, J. Solution Chem. 11 (1982) 793

18. I. Prigogine, N. Trappeniers, V. Mathot, J. Chem. Phys. 21 (1953) 559

19. M.E. Wieser, Pure Appl. Chem. 78 (2006) 2051

20. F. Kohlrausch, W. Hallwachs, Wied. Ann d Phys. 50 (1893) 118

21. V. Dumitrescu, D. Sandulescu, Rev. Roum. Chim. 3 (1998) 183

22. A. Weissberger, Physical methods of Organic Chemistry Interscience Publishers Inc, New York (1959)

23. J. Peleteiro, C.A. Tovar, R. Escudero, E. Carballo, J.L. Legido, L. Romani, Journal of Solution  Chemistry 22 (1993) 1005

24. T.R.C. Thermodynamic Tables-Hydrocarbons, Thermodynamics Research Center, The Texas A & M. University Sistem: College Station, TX (1993)

25. L. Fernandez, E. Perez, J. Ortega, J. Canosa, J. Wisniak, Fluid Phase Equilib. 341 (2013) 105

26. T.M. Aminabhavi, V.B. Patil, M.I. Aralaguppi, H.T.S. Phayde, J. Chem. Eng. Data 41 (1996) 521

27. N.V. Sastry, M.M. Raj,  J. Chem. Eng. Data 41 (1996) 612

28. T.R.C. Thermodynamic Tables Hydrocarbons Supplement 92, Thermodynamic Research Center, Texas A & M. University College Station, Texas A & M. University TX (1988)

29. T.E. Daubert, R.P. Danner, Physical and Thermodynamic Properties of Pure Chemicals: Data Compilation (Hemisphere Publishing Corporation 1989)

30. M.L. Kijevcanin, S.P. Serbanovic, I.R. Radovic, B.D. Djordjevic, A.Z. Tasic, Fluid Phase Equilib. 251 (2007) 78

31. A. Rodriguez, J. Canosa, A. Dominquez, J. Tojo, Fluid Phase Equilib. 216 (2004) 167

32. G. Garcia-Miaja, J. Troncoso, L. Romani, Fluid Phase Equilib. 274 (2008) 59

33. H.E.  Hoga, R.B. Torres, J. Chem. Thermodynamics 43 (2011) 1104

34. P.S. Nikam, M.C. Jadhav, M. Hasan, J. Chem. Eng. Data 40 (1995) 931

35. F.M. Pang, C.E. Seng, T.T. Teng, M.H. Ibrahim, J. Mol. Liq. 136 (2007) 71

36. Z. Shan, A.A. Asfour, Fluid Phase Equilib. 143 (1998) 253

37. A. Chafer, J. de la Torre, E. Lladosa, J.B. Monton, Fluid Phase Equilib. 361 (2014) 23

38. J.A. Riddick, W.B. Bunger, T.K. Sakano, Organic Solvents. Physical Properties and Methods of Purification Techniques of Chemistry, 4th Ed. (Wiley Interscience: New York 1986)

39. S.L. Oswal, H.S. Desai, Fluid Phase Equilib. 149 (1998) 359

40. M.G. Prolongo, R.M. Masegosa, H.I. Fuentes, A. Horta, J. Phys. Chem. 88 (1984) 2163

41. G.P. Dubey, M. Sharma, N. Dubey, J. Chem. Thermodynamics 40 (2008) 309

42. C. Franjo, E. Jimenez, T.P. Iglesias, J.L. Legido, M.I. Paz Andrade, J. Chem. Eng. Data 40 (1995) 68

43. C. Franjo, C.P. Menaut, E. Jimenez, J.L. Legido, M.I. Paz Andrade, J. Chem. Eng. Data 40 (1995) 992

44. L. Romani, J. Peleteiro, T.P. Iglesias, E. Carballo, R. Escudero, J.L. Legido, J. Chem. Eng. Data 39 (1994) 19

45. J. Ortega, M.I. Paz Andrade, E. Rodriguez, J. Chem. Eng. Data 31 (1986) 336

46. R. Mehra, M. Pancholi, Indian Journal of Physics 80 (2006) 253

47. C. Yang, W. Xu, P. Ma, J. Chem. Eng. Data 49 (2004) 1794

48. M.N. Roy, A. Sinha, B. Sinha, J. Solution Chem. 34 (2005) 1311

49. A.C. Gomez-Marigliano, A. Arce, E. Rodil, A. Soto, J. Chem. Eng. Data 55 (2010) 92

50. S. Aznarez, M.M.E.F.D.R. Holgado, E.L. Arancibia, J. Mol. Liq. 124 (2006) 78

51. P.S. Nikam, B.S. Jagdale, A.B. Sawant, M. Hasan, J. Chem. Eng. Data 45 (2000) 214

52. A.R. Mahajan, S.R. Mirgane, Journal of Thermodynamics (2013) doi:10.1155/2013/571918

53. D. Săndulescu, Chimie Fizică 1rd edn. (Ed. Ştiinţifică, Bucureşti 1979)

54. T.M. Letcher, R.C. Baxter, Journal of Solution Chemistry 18 (1989) 65

55. H. Funke, M. Wetzel, A. Heintz, Pure Appl. Chem. 61 (1989) 1429

56. R.B. Torres, A.Z. Francesconi, P.L.O. Volpe, Fluid Phase Equilib. 210 (2003) 287

57. A.J. Treszczanowicz, D. Patterson, G.C. Benson, T. Guttman-Kasprzycka, Fluid Phase Equilib. 50 (1989) 235

58. A. Heintz, Ber Bunsenges Phys. Chem. 89 (1985) 172

59. D. Kulikov, S.P. Verevkin, A. Heintz, Fluid Phase Equilib. 192 (2001) 187

60. E. Bich, D. Papaioannou, A. Heintz, E. Tusel-Langer, R.N. Lichtenthaler, Fluid Phase Equilib. 156 (1999) 115

61. U. Domanska, Fluid Phase Equilib. 130 (1997) 207

62. M. Bender, A. Heintz, Fluid Phase Equilib. 89 (1993) 197

63. J.B. Irving, NEL Report 630 National Eng Lab East Kilbride, UK (1977)

64. J.B. Irving, NEL Report 631 National Eng Lab East Kilbride, UK (1977)

65. L. Grunberg, A.H. Nissan, Nature 164 (1949) 799

66. E.L. Heric, J.G. Brewer, J. Chem. Eng. Data 12 (1967) 574

67. R.A. McAllister, AIChE J. 6 (1960) 427

68. A. Jouyban, M. Khoubnasabjafari, Z. Vaez-Gharamaleki, Z. Fekari, W.E. Jr. Acree, J. Chem. Pharm. Bull 53 (2005) 519

69. A. Jouyban, A. Fathi-Azarbayjani, M. Khoubnasabjafari, W.E. Jr. Acree, Ind. J. Chem. 44 (2005) 1553

70. D.W. Marquardt, J. Soc. Indust. Appl. Math. 11 (1963) 431

71. R.K. Nigam, B.S. Mahl, Indian Journal of Chemistry 9 (1971) 1255

72. H. Eyring, M.S. Jhon, Significant Liquid Structures (John Wiley, New York 1969)

73. M.A. Saleh, S. Akhtar, S. Begum, M.S. Ahmed, S.K. Begum, Physics and Chemistry of Liquids 42 (2004) 615

74. M.E.F. De Ruiz Holgado, C.R. De Schaefer, F. Davolio, M. Katz, Can. J. Chem. 71 (1993) 790

75. T.M. Reed, T.E. Taylor, J. Phys. Chem. 63 (1959) 58

76. R. Meyer, M. Meyer, J. Metzer, A. Peneloux, J. Phys. Chem. 62 (1971) 405.