Densities and viscosities for binary mixtures of n-heptane with alcohols at

different temperatures

MARIA MAGDALENA BUDEANU*, VASILE DUMITRESCU

Chemistry Department, Petroleum and Gas University of Ploiesti, Ploiesti, Romania

*Corresponding author. E-mail: m19magda@yahoo.com

Abstract: Densities (ρ) and viscosities (η) of the binary systems n-heptane with alcohols (ethanol, n-propanol and iso-propanol) were measured at temperatures between 288.15 and 308.15 K and atmospheric pressure, over the whole composition range. The excess values of molar volume (VE) and viscosity (ηE) were calculated from experimental measurements. The excess functions of the binary systems were fitted to Redlich-Kister Equation. Comparison between experimental excess molar volume and that calculated from Flory and Prigogine-Flory-Patterson theories has also been made. Viscosity results were fitted to the equations of Grunberg-Nissan, Heric-Brewer, Jouyban-Acree and McAllister. Also, the activation energies of viscous flow have been obtained and their variations with compositions have been discussed.

Keywords: n-heptane; ethanol; n-propanol; iso-propanol; Flory and Prigogine-Flory-Patterson theories.

RUNNING TITLE:  DENSITIES, VISCOSITIES FOR N-HEPTANE + ALCOHOLS

INTRODUCTION

Experimental liquid densities and viscosities of pure hydrocarbons and their mixtures are useful in design and simulation processes. The physicochemical properties play an important role in the understanding of several industrial processes.1 Therefore, experimental measurements are needed to understand the fundamental behaviour of these properties and then to develop new models.2 The derived properties (excess molar volumes and excess viscosities) in combinations with other mixing properties provide valuable information for qualitatively analyzing the molecular interactions between molecules.3

In this work the densities and viscosities of three binary systems of n-heptane + ethanol or + n-propanol or + iso-propanol were measured at temperatures between 288.15 and 308.15 K, over the entire composition range and at atmospheric pressure. From these experimental data, excess molar volumes and excess viscosities were calculated and correlated by the Redlich-Kister Equation.

The experimental data of excess volumes of the binary mixtures were used to test to applicability of Flory4-7 and Prigogine-Flory-Patterson8,9 theories, wich has not previously been employed for the study of excess molar volume of binary mixtures of ethanol, n-propanol and iso-propanol with n-heptane. The thermodynamic functions of activation of viscous flow have been estimated from the experimental data. The purpose of this paper is also to test different semiempirical equations to correlate viscosity of binary mixtures.

EXPERIMENTAL

Materials

The chemicals n-heptane (mole fraction purity > 0.990) was supplied by Sigma-Aldrich, n-propanol (mole fraction purity > 0.999) was obtained from Merck, ethanol (mole fraction purity > 0.993) and iso-propanol (mole fraction purity > 0.997) were supplied by Chemical Company. The chemicals were dried over molecular sieves (Fluka type 4 Å). The purity was checked through chromatographic analysis. Samples were determined by weighing at atmospheric pressure and ambient temperature using a Adventurer Pro AV 264CM balance with an uncertainty of and precision of ± 10-4 g. The error in the final mole fraction is estimated to be less than ± 0.0001. Conversion to molar quantities was based on the relative atomic mass table of 2006 issued by I.U.P.A.C.10

Measurements

The densities were determined by hydrostatic weighing method of Kohlrausch11 with the precision of  ± 0.05 kg m-3. The experimental technique has been previously described12, an ultra-thermostat type U10 (Freital) has been used to maintain a constant temperature (± 0.05 K).

The uncertainty in the excess molar volume is estimated to be ± 2x10-9 m3.mol-1. Viscosities of the pure compounds and of the binary mixtures were determined with an Ubbelohde kinematic viscometer13, Viscosity Measuring Unit ViscoClock (Schott-Gerate GmbH), that was kept in a vertical position in a water thermostat. A thermostatically controlled bath (U10 constant to

± 0.05 K) was used. The kinematic viscosity was calculated using the relation (1):

ν=At−Bt 	(1)

where t / s is the flowing time of a constant volume liquid through the viscometer capillary. Accuracy of time measurement for the Viscosity Measuring Unit ViscoClock is ± 0.01s. A and B are characteristic constants of the used viscometer, which were determined by taking bidistilled water and benzene (Merck, mole fraction purity > 0.995) as the calibrating liquids for correction of kinetic energy deviations. The dynamic viscosity was determined from the Eq. (2):

η=νρ 	(2)

where ρ is the density of the liquid. The precision of the viscosity was estimated to be ± 0.001 mPa.s. In all determinations, triplicate experiments were performed at each composition and temperature, and the arithmetic mean was taken for the calculations of the viscosity. The uncertainty in the excess viscosity is estimated to be ± 2x10-4 mPa.s.

RESULT AND DISCUSSION

The measured densities and viscosities of the pure component liquids present good agreement with the literature values, as seen in Table I. Densities and viscosities of the binary mixtures of n-heptane + ethanol, n-heptane + n-propanol and n-heptane + iso-propanol are reported in Table II. The results of this study are in close agreement with the values reported by different authors.14-20

In the literature, Jimenez et al.14 presented the values of viscosities and densities for

n-propanol + n-heptane at 293.15, 298.15, 303.15 and 308.15 K. Keller et al.15 investigated the excess molar volumes of n-propanol + n-heptane at 298.15 K and Tanaka et al.16 reported excess molar volumes of propan-2-ol + n-heptane at the temperature 298.15 K. Pereiro et al.17 studied densities for ethanol + n-heptane at 293.15, 298.15, 303.15 K and Orge et al.18 reported densities and viscosities for ethanol + n-heptane and n-propanol + n-heptane at 298.15 K. Papaioannou et al.19 presented densities for n-heptane + ethanol at 298.15 K and Zeberg-Mikkelsen et al.20 investigated dynamic viscosities for ethanol + n-heptane at 293.15, 313.15, 333.15 and 353.15K.

Figures S-1. – S-4. of the Supplementary material to this work includes comparisons of our experimental density and viscosity of n-heptane + ethanol and n-heptane + n-propanol at 298.15 K with data available in the literature.

TABLE I. Comparison of experimental densities (ρ) and viscosities (η) of pure liquids with literature values at different temperature and atmospheric pressure

| Component | T / K | ρ / kg m-3 |  | η / mPa s |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Experimental | Literature | Experimental | Literature |
| n-Heptane | 288.15 | 688.20 | 687.921 | 0.4427 | - |
|  | 293.15 | 684.30 | 683.922 | 0.4197 | 0.416926 |
|  | 298.15 | 680.00 | 679.4823 | 0.3934 | 0.390025 |
|  | 303.15 | 675.70 | 675.124 | 0.3774 | 0.378526 |
|  | 308.15 | 671.40 | 670.925 | 0.3582 | 0.347027 |
| Ethanol | 288.15 | 795.80 | - | 1.3652 | - |
|  | 293.15 | 789.50 | 789.4528 | 1.2206 | 1.198831 |
|  | 298.15 | 785.70 | 785.029 | 1.1088 | 1.105029 |
|  | 303.15 | 781.60 | 781.030 | 1.0056 | 1.010232 |
|  | 308.15 | 777.00 | 776.730 | 0.9222 | 0.903731 |
| n-Propanol | 288.15 | 808.41 | - | 2.4915 | - |
|  | 293.15 | 804.40 | 804.2833 | 2.2304 | 2.197033 |
|  | 298.15 | 800.71 | 800.2133 | 1.9885 | 1.970029 |
|  | 303.15 | 796.71 | 796.4233 | 1.7641 | 1.784334 |
|  | 308.15 | 792.90 | 792.2733 | 1.5540 | 1.546034 |
| iso-Propanol | 288.15 | 791.20 | - | 2.7622 | - |
|  | 293.15 | 787.11 | 785.3533 | 2.4064 | 2.414033 |
|  | 298.15 | 783.20 | 782.7035 | 2.0546 | 2.043636 |
|  | 303.15 | 778.91 | 777.1233 | 1.7908 | 1.785033 |
|  | 308.15 | 774.51 | 772.8833 | 1.5605 | 1.551037 |

*Uncertainties, u, are: u(x)= ±0.0001; u(T)= ± 0.05 K; u(ρ)= ±0.05 kg.m-3; u(t)= ± 0.01s; u(η)= ± 0.001 mPa.s.

TABLE II. Densities (ρ) and viscosities (η) of the (n-heptane + alcohols) mixtures at different temperatures and atmospheric pressure

| x | ρ / kg m-3 |  |  |  |  | η / mPa s |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T / K |  |  |  |  | T / K |  |  |  |  |
|  | 288.15 | 293.15 | 298.15 | 303.15 | 308.15 | 288.15 | 293.15 | 298.15 | 303.15 | 308.15 |
| (x) n-heptane + (1-x) ethanol |  |  |  |  |  |  |  |  |  |  |
| 0.1005 | 770.87 | 764.80 | 760.84 | 756.43 | 751.70 | 1.0803 | 0.9853 | 0.9069 | 0.8186 | 0.7535 |
| 0.2020 | 751.33 | 745.76 | 741.64 | 737.40 | 732.87 | 0.8992 | 0.8256 | 0.7656 | 0.6950 | 0.6432 |
| 0.3022 | 736.60 | 731.34 | 727.22 | 722.84 | 718.25 | 0.7824 | 0.7149 | 0.6673 | 0.6164 | 0.5695 |
| 0.4016 | 725.02 | 720.14 | 716.01 | 711.49 | 706.93 | 0.6870 | 0.6285 | 0.5856 | 0.5429 | 0.5077 |
| 0.5079 | 715.08 | 710.58 | 706.41 | 701.84 | 697.37 | 0.5992 | 0.5670 | 0.5214 | 0.4865 | 0.4521 |
| 0.5967 | 708.24 | 703.81 | 699.64 | 695.07 | 690.62 | 0.5510 | 0.5160 | 0.4815 | 0.4506 | 0.4271 |
| 0.7041 | 701.43 | 697.06 | 692.83 | 688.47 | 684.00 | 0.5002 | 0.4658 | 0.4422 | 0.4154 | 0.3957 |
| 0.8008 | 696.14 | 691.94 | 687.63 | 683.35 | 678.97 | 0.4629 | 0.4428 | 0.4149 | 0.3953 | 0.3691 |
| 0.8935 | 691.89 | 687.76 | 683.40 | 679.12 | 674.67 | 0.4485 | 0.4230 | 0.4021 | 0.3827 | 0.3633 |

TABLE II. continued

| x | ρ / kg m-3 |  |  |  |  | η / mPa s |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T / K |  |  |  |  | T / K |  |  |  |  |
|  | 288.15 | 293.15 | 298.15 | 303.15 | 308.15 | 288.15 | 293.15 | 298.15 | 303.15 | 308.15 |
| (x) n-heptane + (1-x) n-propanol |  |  |  |  |  |  |  |  |  |  |
| 0.1020 | 785.75 | 781.54 | 777.40 | 773.16 | 769.05 | 1.8182 | 1.6409 | 1.4771 | 1.3187 | 1.1889 |
| 0.2018 | 767.57 | 763.40 | 759.30 | 754.91 | 750.70 | 1.4201 | 1.2906 | 1.1663 | 1.0466 | 0.9659 |
| 0.2990 | 752.27 | 748.15 | 744.01 | 739.72 | 735.47 | 1.1367 | 1.0521 | 0.9499 | 0.8669 | 0.7776 |
| 0.4005 | 738.89 | 734.60 | 730.49 | 726.12 | 721.85 | 0.9138 | 0.8426 | 0.7687 | 0.7025 | 0.6444 |
| 0.5017 | 727.19 | 722.99 | 718.84 | 714.35 | 710.15 | 0.7616 | 0.6995 | 0.6453 | 0.5949 | 0.5454 |
| 0.6033 | 716.75 | 712.60 | 708.40 | 704.04 | 699,76 | 0.6327 | 0.5899 | 0.5511 | 0.5056 | 0.4781 |
| 0.7025 | 708.19 | 704.08 | 699.87 | 695.47 | 691.09 | 0.5474 | 0.5126 | 0.4886 | 0.4543 | 0.4308 |
| 0.8010 | 700.67 | 696.47 | 692.26 | 687.94 | 683.53 | 0.4997 | 0.4793 | 0.4419 | 0.4254 | 0.3988 |
| 0.8918 | 694.39 | 690.34 | 686.11 | 681.70 | 677.38 | 0.4744 | 0.4475 | 0.4174 | 0.3943 | 0.3783 |
| (x) n-heptane + (1-x) iso-propanol |  |  |  |  |  |  |  |  |  |  |
| 0.1003 | 772.03 | 767.65 | 763.67 | 759.16 | 754.60 | 1.8249 | 1.5964 | 1.4129 | 1.2300 | 1.0913 |
| 0.2009 | 756.15 | 752.02 | 747.94 | 743.53 | 738.83 | 1.3311 | 1.1776 | 1.0444 | 0.9270 | 0.8401 |
| 0.3009 | 742.52 | 738.36 | 734.19 | 729.84 | 725.32 | 1.0146 | 0.9242 | 0.8165 | 0.7388 | 0.6728 |
| 0.3980 | 731.65 | 727.47 | 723.08 | 718.63 | 714.21 | 0.8246 | 0.7437 | 0.6851 | 0.6200 | 0.5770 |
| 0.5002 | 721.62 | 717.13 | 712.92 | 708.42 | 703.97 | 0.6801 | 0.6223 | 0.5866 | 0.5333 | 0.4915 |
| 0.5961 | 713.33 | 708.85 | 704.65 | 700.17 | 695.74 | 0.5870 | 0.5479 | 0.5025 | 0.4679 | 0.4441 |
| 0.6981 | 705.59 | 701.17 | 696.97 | 692.49 | 688.08 | 0.5243 | 0.4930 | 0.4623 | 0.4351 | 0.4083 |
| 0.8014 | 698.89 | 694.59 | 690.12 | 685.66 | 681.21 | 0.4845 | 0.4607 | 0.4268 | 0.4137 | 0.3805 |
| 0.9084 | 692.49 | 688.42 | 684.05 | 679.59 | 675.25 | 0.4559 | 0.4360 | 0.4025 | 0.3897 | 0.3637 |

The excess molar volumes were calculated from the densities of the pure liquids and their mixtures using the following Eq. (3):

VE=xM1+1−xM2ρ−xM1ρ1+1−xM2ρ2 	(3)

where x and (1-x) are the mole fraction of the components, M1 and M2 are the molecular masses of the components 1 and 2, and ρ, ρ1 and ρ2 are the respective densities of the solution and of the pure components.

The experimental values of viscosity (η) are used to calculate the excess viscosity (ηE) defined by the Eq. (4):

ηE=η−xη1+1−xη2 	(4)

where η1 and η2 are the viscosities of pure components.

The results of VE and ηE are reported in Tables S-I and S-II of the Supplementary material.

The excess functions of the binary systems can be represented by a Redlich-Kister type Eq. (5):

XE=xixjAk2xi−1k 	(5)

where XE represents any of the following properties: VE, ηE and xi, xj are the mole fractions of the components i and j, respectively, and k is the number of  the polynomial coefficient Ak.

The values of these coefficients are indicated in Tables III and IV along with the standard deviation, σ, defined by relation:

σ=XexpE−XcalcE2m−n1/2 	(6)

where XexpE is the experimental excess function, XcalcE is the calculated excess function with Eq. (5),  m is the number of data points and n is the number of estimated parameters.

TABLE III. Coefficients Ak and standard deviations, σ, for excess molar volume at different temperatures

| T / K | 106×A0 / m3 mol-1 | 106×A1 / m3 mol-1 | 106×A2 / m3 mol-1 | 106×A3 / m3 mol-1 | 106× σ / m3 mol-1 |
| --- | --- | --- | --- | --- | --- |
| (x) n-heptane + (1-x) ethanol |  |  |  |  |  |
| 288.15 | 1.72482 | -0.04426 | 0.30956 | 1.24112 | 0.0078 |
| 293.15 | 1.76419 | -0.11136 | 0.58527 | 1.12746 | 0.0062 |
| 298.15 | 1.79120 | -0.17632 | 0.83641 | 1.51667 | 0.0067 |
| 303.15 | 1.97390 | -0.08814 | 0.55322 | 1.13746 | 0.0132 |
| 308.15 | 2.04506 | -0.11059 | 0.6910 | 1.27051 | 0.0187 |
| (x) n-heptane + (1-x) n-propanol |  |  |  |  |  |
| 288.15 | 0.92022 | 0.49895 | 0.42104 | -0.08171 | 0.0151 |
| 293.15 | 1.03145 | 0.52803 | 0.52025 | -0.10929 | 0.0122 |
| 298.15 | 1.08665 | 0.54794 | 0.75585 | -0.38737 | 0.0148 |
| 303.15 | 1.19777 | 0.52584 | 0.87876 | -0.36976 | 0.0163 |
| 308.15 | 1.26477 | 0.55144 | 1.14112 | -0.47573 | 0.0116 |
| (x) n-heptane + (1-x) iso-propanol |  |  |  |  |  |
| 288.15 | 1.17179 | -0.10704 | 0.49217 | 0.81795 | 0.0156 |
| 293.15 | 1.39784 | 0.45763 | 0.46907 | -0.17666 | 0.0184 |
| 298.15 | 1.48027 | 0.30581 | 0.74264 | 0.65875 | 0.0151 |
| 303.15 | 1.5811 | 0.4509 | 0.95112 | 0.56564 | 0.0204 |
| 308.15 | 1.65291 | 0.47941 | 1.22512 | 0.3723 | 0.0159 |

TABLE IV. Coefficients Ak and standard deviations, σ, for excess viscosities at different temperatures

| T / K | A0 / mPa s | A1 / mPa s | A2 / mPa s | A3 / mPa s | σ / mPa s |
| --- | --- | --- | --- | --- | --- |
| (x) n-heptane + (1-x) ethanol |  |  |  |  |  |
| 288.15 | -1.17059 | 0.44174 | -0.56711 | 0.43989 | 0.0036 |
| 293.15 | -1.01004 | 0.41839 | -0.42647 | 0.18487 | 0.0027 |
| 298.15 | -0.89479 | 0.3403 | -0.27042 | 0.17447 | 0.0018 |

TABLE IV. continued

| T / K | A0 / mPa s | A1 / mPa s | A2 / mPa s | A3 / mPa s | σ / mPa s |
| --- | --- | --- | --- | --- | --- |
| (x) n-heptane + (1-x) ethanol |  |  |  |  |  |
| 303.15 | -0.79589 | 0.27399 | -0.30653 | 0.29205 | 0.0027 |
| 308.15 | -0.71489 | 0.27151 | -0.29219 | 0.20123 | 0.0035 |
| (x) n-heptane + (1-x) n-propanol |  |  |  |  |  |
| 288.15 | -2.82496 | 1.22654 | -0.9919 | 1.09367 | 0.0059 |
| 293.15 | -2.48186 | 1.00314 | -0.79575 | 1.08458 | 0.0023 |
| 298.15 | -2.16384 | 0.97194 | -0.70223 | 0.6812 | 0.0056 |
| 303.15 | -1.89223 | 0.78527 | -0.59003 | 0.72463 | 0.0022 |
| 308.15 | -1.62645 | 0.71789 | -0.30942 | 0.30673 | 0.0063 |
| (x) n-heptane + (1-x) iso-propanol |  |  |  |  |  |
| 288.15 | -3.65753 | 2.28763 | -2.05348 | 1.63074 | 0.0105 |
| 293.15 | -3.12076 | 1.91927 | -1.75579 | 1.58995 | 0.0117 |
| 298.15 | -2.56338 | 1.63908 | -1.3988 | 0.90719 | 0.0054 |
| 303.15 | -2.19627 | 1.36686 | -1.17012 | 1.01654 | 0.0063 |
| 308.15 | -1.84071 | 1.11945 | -1.01897 | 0.75203 | 0.0061 |

Several effects may contribute to the values of excess molar volumes, such as breaking of liquid order on mixing, unfavorable interaction between groups, differences in molecular volumes and differences in free volumes between liquids components.38

The experimental excess molar volumes for all the binary mixtures studied in this work are positive in the whole composition range and at all temperatures. The positive values are due to expansion of solution volume due to mixing caused by the hydrogen bond rupture and dispersive interactions between unlike molecules. In this sense, the addition of non-polar solvent causes disruption of alcohol aggregates through the breaking of hydrogen bonds, making VE positive, since aggregates have higher volumes than the sum of their components.39

The curves for all binary systems are asymmetric, with its maximum displace toward a high mole fraction of n-heptane, this behavior agrees with the literature.40-42 The excess molar volumes decreases with increasing alkyl chain length of the alcohol. Ortega et al.43 showed that VE values are greater when alkanes are mixed with secondary or tertiary alcohols than when mixed with primary alcohols. The effect of temperature on the VE shows a systematic decrease with temperature for all the mixtures. As an illustration, Fig. 1 shows experimental (Eq. 3) and calculated values (Eq. 5) of VEat 298.15 K constant temperature.

The excess viscosities values may be generally explained by considering the difference in size and shape of the component molecules and specific interactions between unlike molecules such as H-bond formation and charge transfer complexes may cause increase in viscosity in mixtures rather than in pure component.44 Positive values of excess viscosities are indicate of strong interactions, while negative values indicate weaker interactions.45

The excess viscosities are negative over the entire range of mole fractions at all the temperatures. The negative excess viscosity was explained by many authors through different forms.46-49 The negative values support the main factor of breaking of the self-associated alcohols and weak interactions between unlike molecules.50 The minima of ηE-x curves occur at low mole fraction of n-heptane. This behavior is characteristic of mixtures of a substance that exhibits molecular association and another that does not.14 If the temperature increases the negative values of excess viscosity decrease. This can be explained by breaking hydrogen bonds and increasing mobility of molecules due of the increase of thermal energy.31 Figure 2 shows experimental (Eq. 4) and calculated values (Eq. 5) of ηE at 303.15 K constant temperature.

Fig. 1. Experimental excess molar volumes versus mole fraction x at 298.15 K for the mixtures:

(■) (x) n-heptane + ethanol; (▲) (x) n-heptane + n-propanol; ()  (x) n-heptane + iso-propanol.

Continuous curve was calculated from Eq. (5) for experimental data.

Fig. 2. Experimental excess viscosities versus mole fraction x at 303.15 K for the mixtures :

(■) (x) n-heptane + ethanol; (▲) (x) n-heptane + n-propanol; ()  (x) n-heptane + iso-propanol.

Continuous curve was calculated from Eq. (5) for experimental data.

Flory and Prigogine-Flory-Patterson theories

Experimental results of the molar excess volume have been used to test the applicability of the Flory4-7 and Prigogine-Flory-Patterson (PFP) theories.8, 9

In the Flory theory, the excess molar volume is related to the reduced volumev, the reduced temperature Tand the adjustable interactional parameterχ12. The PFP theory includes three contributions in order to explain the thermodynamic behavior of the liquid mixtures: an interactional contribution VintE, which is proportional to the interactional parameter χ12, the free volume contribution VFVE, which arises from the dependence of the reduced volume upon the reduced temperature as a result of the difference between the degree of expansion of the two components and the internal pressure contribution VpE, which depends on both the difference of characteristic pressures and the reduced volumes of the components.

The excess molar volume given by the Flory theory is:

VE=x1V1+x2V2ϕ1ν1+ϕ2ν273T−T043−ϕ1ν1+ϕ2ν213 	(7)

The Prigogine Flory Patterson Equation is:

VEx1V1+x2V2=ν13−1ν2343ν−13−1ψ1θ2χ12p1−ν1−ν22149ν−13−143ν−13−1νψ1ψ2+ν1−ν2p1−p2ψ1p2+ψ2p1ψ1ψ2=VintEx1V1+x2V2+VFVEx1V1+x2V2+VpEx1V1+x2V2 	(8)

where:

vis the reduced volume:

ν=αT31+αT+13 	(9)

V- characteristic volume;

p∗- characteristic pressure;

T- reduced temperature:

T=ϕ1p1T1+ϕ2p2T2ϕ1p1+ϕ2p2−ϕ1θ2χ12 	(10)

T0- ,,ideal” reduced temperature for the mixture;

ψi- contact energy fraction:

ψ1=1−ψ2=ϕ1p1/ϕ1p1+ϕ2p2 		(11)

ϕi- hard core volume fraction:

ϕ2=1−ϕ1=x2V2/x1V1+x2V2                                                (12)

θi- surface site fraction:

θ2=1−θ1=ϕ2V213/ϕ1V113+ϕ2V213                            (13)

The coefficient of thermal expansion, α, was obtained from the following equation: 51

α=1/V∂V/∂Tp,                                                         (14)

where:

V  is the molar volume of the solution;

T - temperature.

The various parameters involved in Eqs. 7 and 8 for the pure components and the mixture are obtained from Flory’s theory4, 5 and are shown in Tables V and VI.

The values of the interaction parameter 12, were derived by fitting the theory to experimental values of the excess molar volume for equimolar solutions. The results of VE calculated from Eqs. 7 and 8 are reported in Table VII. These results were compared by means of the percentage absolute average deviation (ADD) with the following equation:

ADD%=VexpE−VcalcEVexpE.100 	(15)

TABLE V. Parameters of the pure components, coefficient of thermal expansion α, isothermal compressibility kT, reduced volume v, characteristic volume V*, characteristic pressure P*, characteristic temperature T*, and reduced temperature T for Flory theory

| Component | 103.α / K-1 |  | kT / 104.MPa-1 | v | 106.V* / m3 mol-1 | 106.p*/ J m-3 | T*/ K | T |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 293.15 K |  |  |  |  |  |  |  |  |
| n-Heptane |  | 1.24 | 14.3852 | 1.2909 | 113.4584 | 421.0756 | 4638.1 | 0.0632 |
| Ethanol |  | 1.19 | 11.0553 | 1.2815 | 45.5330 | 518.5073 | 4733.6 | 0.0619 |
| n-Propanol |  | 0.97 | 9.5554 | 1.2381 | 60.3438 | 456.4552 | 5281.1 | 0.0555 |
| iso-Propanol |  | 1.07 | 10.8154 | 1.2583 | 60.6829 | 459.4142 | 5003.3 | 0.0586 |
| 298.15 K |  |  |  |  |  |  |  |  |
| n-Heptane |  | 1.24 | 14.60655 | 1.2948 | 113.8119 | 424.2082 | 4678.1 | 0.0637 |
| Ethanol |  | 1.19 | 11.5356 | 1.2854 | 45.6163 | 508.4346 | 4773.5 | 0.0624 |
| n-Propanol |  | 0.97 | 10.2657 | 1.2415 | 60.4575 | 434.4775 | 5320.4 | 0.0560 |
| iso-Propanol |  | 1.07 | 11.3056 | 1.2619 | 60.8110 | 449.5515 | 5042.9 | 0.0591 |
| 303.15 K |  |  |  |  |  |  |  |  |
| n-Heptane |  | 1.24 | 16.353 | 1.2987 | 114.1912 | 388.835 | 4718.2 | 0.0642 |
| Ethanol |  | 1.19 | 11.9553 | 1.2892 | 45.7197 | 501.7603 | 4813.5 | 0.0630 |
| n-Propanol |  | 0.97 | 10.5754 | 1.2449 | 60.5970 | 431.1326 | 5359.7 | 0.0566 |
| iso-Propanol |  | 1.07 | 11.8258 | 1.2655 | 60.9737 | 439.4645 | 5082.5 | 0.0596 |
| 308.15 K |  |  |  |  |  |  |  |  |
| n-Heptane |  | 1.24 | 17.759 | 1.3026 | 114.5799 | 366.1661 | 4758.4 | 0.0647 |
| Ethanol |  | 1.19 | 12.360 | 1.2930 | 45.8555 | 498.4433 | 4853.5 | 0.0635 |
| n-Propanol |  | 0.97 | 11.0831 | 1.2482 | 60.7244 | 420.3194 | 5399.0 | 0.0571 |
| iso-Propanol |  | 1.07 | 12.3231 | 1.2690 | 61.1482 | 430.9963 | 5122.2 | 0.0602 |

TABLE VI. Parameters of the liquid mixtures for Flory and Prigogine-Flory-Patterson theories

| T / K |  |  | v | T | 12.106 / J m-3 | 1 |
| --- | --- | --- | --- | --- | --- | --- |
| (x) n-heptane + (1-x) ethanol |  |  |  |  |  |  |
| 293.15 | 0.2864 | 0.2284 | 1.2938 | 0.0636 | 34.9794 | 0.6692 |
| 298.15 | 0.2861 | 0.2281 | 1.2977 | 0.0641 | 34.3676 | 0.6755 |
| 303.15 | 0.2859 | 0.2278 | 1.3022 | 0.0647 | 35.3985 | 0.6593 |
| 308.15 | 0.2858 | 0.2277 | 1.3062 | 0.0652 | 34.9129 | 0.6473 |
| (x) n-heptane + (1-x) n-propanol |  |  |  |  |  |  |
| 293.15 | 0.3472 | 0.3012 | 1.2755 | 0.0611 | 25.2022 | 0.6343 |
| 298.15 | 0.3469 | 0.3008 | 1.2794 | 0.0616 | 21.7395 | 0.6476 |
| 303.15 | 0.3467 | 0.3005 | 1.2835 | 0.0622 | 26.2681 | 0.6296 |
| 308.15 | 0.3464 | 0.3001 | 1.2874 | 0.0627 | 27.4530 | 0.6217 |

TABLE VI. continued

| T / K |  |  | v | T | 12.106 / J m-3 | 1 |
| --- | --- | --- | --- | --- | --- | --- |
| (x) n-heptane + (1-x) iso-propanol |  |  |  |  |  |  |
| 293.15 | 0.3485 | 0.3027 | 1.2835 | 0.0622 | 24.7486 | 0.6315 |
| 298.15 | 0.3482 | 0.3024 | 1.2876 | 0.0627 | 24.2387 | 0.6385 |
| 303.15 | 0.3481 | 0.3022 | 1.2917 | 0.0633 | 25.8128 | 0.6236 |
| 308.15 | 0.3480 | 0.3021 | 1.2956 | 0.0638 | 26.4277 | 0.6142 |

TABLE VII. Experimental and calculated excess volumes at 0.5 mole fraction and calculated values of the three contributions to VE

| T / K | 106.VEexp. / m3 mol-1 | 106.VEFlory/ m3 mol-1 | 106.VEPFP / m3 mol-1 | 106.VEint. / m3 mol-1 | 106.VEFV / m3 mol-1 | 106.VEp* / m3 mol-1 | ADD% Flory | ADD% PFP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (x) n-heptane + (1-x) ethanol |  |  |  |  |  |  |  |  |
| 293.15 298.15 303.15 308.15 | 0.4410 0.4478 0.4935 0.5113 | 0.4374 0.4441 0.4890 0.5065 | 0.4453 0.4521 0.4988 0.5172 | 0.4805 0.4832 0.5429 0.5712 | 0.0023 0.0023 0.0024 0.0025 | -0.0329 -0.0288 -0.0417 -0.0515 | 0.8 0.8 0.9 0.9 | 0.5 0.9 1.0 1.1 |
| (x) n-heptane + (1-x) n-propanol |  |  |  |  |  |  |  |  |
| 293.15 298.15 303.15 308.15 | 0.2579 0.2717 0.2994 0.3162 | 0.2567 0.2704 0.2979 0.3145 | 0.2624 0.2756 0.3050 0.3228 | 0.4304 0.3847 0.5044 0.5650 | 0.0832 0.0838 0.0876 0.0903 | -0.0848 -0.0253 -0.1118 -0.1519 | 0.5 0.5 0.5 0.5 | 1.7 1.4 1.9 2.1 |
| (x) n-heptane + (1-x) iso-propanol |  |  |  |  |  |  |  |  |
| 293.15 298.15 303.15 308.15 | 0.3495 0.3701 0.3953 0.4132 | 0.3474 0.3677 0.3926 0.4104 | 0.3531 0.3737 0.4000 0.4189 | 0.4419 0.4443 0.5161 0.5653 | 0.0319 0.0324 0.0337 0.0347 | -0.0569 -0.0382 -0.0824 -0.1117 | 0.6 0.6 0.7 0.7 | 1.0 1.0 1.8 1.4 |

The results in Table VII reveal that ADD in the predictions of excess molar volume for

n-heptane + ethanol mixtures are less than 0.9% using the Flory theory and less than 1.1% using Prigogine-Flory-Patterson theory. For n-heptane + n-propanol system the ADD value is 0.5% using the Flory theory and  less than 2.0 % using Prigogine-Flory-Patterson theory, while for

n-heptane + iso-propanol system the ADD values are less than 0.7% for Flory theory and less than 1.4 % using Prigogine-Flory-Patterson theory. These low values of percentage absolute average deviation obtained suggest the validity of these theories. An analysis of each of the three contributions to excess molar volume shows that the interaction contribution is positive, being the most important contribution. The volume contribution is positive, while the internal pressure contribution is negative.

The viscosity data correlation

Several empirical and semi-empirical relations have been used to represent the dependence of viscosity on concentration of components in binary systems. These relations are classified according to the number of adjustable parameters used to account for the deviation from some average.61, 62

Grunberg-Nissan63 proposed the following equation based on one parameter:

lnη=x1lnη1+x2lnη2+x1x2d 	(16)

Heric-Brewer64 expression with two parameters is:

lnη=x1lnη1+x2lnη2+x1lnM1+x2lnM2−lnx1M1+x2M2+x1x2α12+α21x1−x2 (17)

Jouyban Acree65,66 Equation with four parameters is:

lnη=x1lnη1+x2lnη2+x1x2Ajx1−x2jT, where n=3 	(18)

In the Eqs. 16-18 the dynamic viscosity of the liquid mixture is designated by η and η1, η2 are the dynamic viscosities, x1, x2 are the mole fractions of the pure components 1 and 2 constituting the liquid mixture, M1, M2 are the molecular masses, T is the temperature; d, α12,  and Aj are interaction parameters (viscosity coefficients) and reflect the non-ideality of the system.

We tested also the three body McAllister Equation: 67

lnη=x13lnη1+3x12x2lnη12+3x1x22lnη21+x23lnη2−lnx1+x2M2M1+3x12x2ln2+M2M13+3x1x22ln1+2M2M13+x23lnM2M1 	(19)

and the four body McAllister Equation:

lnη=x14lnη1+4x13x2lnη1112+6x12x22lnη1122+4x1x23lnη2221+x24lnη2−lnx1+x2M2M1+4x13x2ln3+M2M14+6x12x22ln1+M2M12+4x1x23ln1+3M2M14+x24lnM2M1 	(20)

The McAllister model adjustable parameters are given by η12, η21, η1112, η1122 and η2221. The parameters that appear in Eqs. 16-20 were estimated using the experimental viscosity data and a non-linear regression analysis employing the Levenberg-Marquardt algorithm.68 Table VIII shows the parameters calculated and the standard deviations between experimental values and those obtained using the semi-empirical relations. The interactional parameter d is negative for binary systems. Nigam and Mahl69 concluded from the study of binary mixtures that if ηE< 0,

d < 0 and magnitude of both are large then the dispersion force would be dominant. Use of three and four parameter equation reduces the σ values significantly. The data show that the Jouyban-Acree model is suitable for 288.15 K, 298.15 K and 303.15 K, while the four body McAllister model is adequate at 293.15 K and 308.15 K for n-heptane and ethanol system. The Jouyban-Acree model shows the best agreement with experimental data for n-heptane + n-propanol system at all temperatures. A comparison of the calculated and the experimental viscosities for the n-heptane + iso-propanol mixtures shows that the Jouyban-Acree model produces accurate results for 288.15 K, 293.15 K, 303.15 K and 308.15 K, while for 298.15 K the four body McAllister model is able to describe viscosities. From this study, it can be concluded that the correlating ability significantly improves for these non-ideal systems as number of adjustable parameters is increased.

TABLE VIII. Parameters for the semi-empirical relations of Grunberg-Nissan, Heric-Brewer, Joyban-Acree and McAllister and standard deviations at different temperatures

| Equation | Parameters and σ | T / K |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 288.15 | 293.15 | 298.15 | 303.15 | 308.15 |
| (x) n-heptane + (1-x) ethanol |  |  |  |  |  |  |
| Grunberg-Nissan | d | -1.03339 | -0.98234 | -0.92819 | -0.93839 | -0.92373 |
|  | σ | 0.0159 | 0.0133 | 0.0225 | 0.0113 | 0.0140 |
| Heric-Brewer | α12 | -0.73607 | -0.68501 | -0.63085 | -0.64117 | -0.62658 |
|  |  | -0.02572 | -0.0298 | -0.03183 | -0.00191 | -0.01579 |
|  | σ | 0.0158 | 0.0130 | 0.0060 | 0.0097 | 0.0121 |
| Jouyban-Acree | A0 | -280.2367 | -273.37925 | -269.89168 | -274.10778 | -272.5385 |
|  | A1 | -7.37491 | 13.79187 | 4.40707 | -4.69605 | 16.13374 |
|  | A2 | -123.0185 | -102.42284 | -48.25045 | -73.12746 | -85.46377 |
|  | A3 | 67.78576 | 0.76301 | 27.02307 | 82.8945 | 37.86172 |
|  | σ | 0.0059 | 0.0066 | 0.0035 | 0.0046 | 0.0091 |
| McAllister three body model | η12 | 0.47063 | 0.4411 | 0.41952 | 0.39759 | 0.37708 |
|  | η21 | 0.68907 | 0.6426 | 0.59851 | 0.5457 | 0.50566 |
|  | σ | 0.0158 | 0.0139 | 0.0189 | 0.0308 | 0.0121 |
| McAllister four body model | η1112 | 0.42355 | 0.40301 | 0.39913 | 0.37311 | 0.35117 |
|  | η1122 | 0.65931 | 0.60454 | 0.5245 | 0.50242 | 0.47761 |
|  | η2221 | 0.74193 | 0.69182 | 0.67092 | 0.60061 | 0.55073 |
|  | σ | 0.0070 | 0.0060 | 0.0035 | 0.0062 | 0.0085 |

TABLE VIII. continued

| Equation | Parameters and σ | T / K |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 288.15 | 293.15 | 298.15 | 303.15 | 308.15 |
| (x) n-heptane + (1-x) n-propanol |  |  |  |  |  |  |
| Grunberg-Nissan | d | -1.33517 | -1.30165 | -1.26286 | -1.27154 | -1.2067 |
|  | σ | 0.0139 | 0.0146 | 0.0076 | 0.0121 | 0.0087 |
| Heric-Brewer | α12 | -1.20515 | -1.17167 | -1.13303 | -1.14173 | -1.07688 |
|  |  | -0.08064 | -0.06412 | -0.01719 | -0.01409 | -0.00629 |
|  | σ | 0.0139 | 0.0151 | 0.0081 | 0.0129 | 0.0093 |
| Jouyban-Acree | A0 | -379.36536 | -380.59804 | -372.73156 | -384.13063 | -379.38218 |
|  | A1 | -87.51838 | -95.69916 | -38.5322 | -56.15547 | -20.57744 |
|  | A2 | -36.79759 | -6.70406 | -26.25663 | -9.42309 | 51.85301 |
|  | A3 | 214.3258 | 252.40447 | 121.40922 | 177.40899 | 76.24936 |
|  | σ | 0.0073 | 0.0078 | 0.0046 | 0.0106 | 0.0068 |
| McAllister three body model | η12 | 0.49922 | 0.47214 | 0.44782 | 0.41777 | 0.39624 |
|  | η21 | 0.93389 | 0.857 | 0.77478 | 0.70271 | 0.64677 |
|  | σ | 0.0140 | 0.0151 | 0.0081 | 0.0129 | 0.0093 |
| McAllister four body model | η1112 | 0.47098 | 0.45576 | 0.42505 | 0.40418 | 0.39999 |
|  | η1122 | 0.71316 | 0.63831 | 0.60604 | 0.54545 | 0.47232 |
|  | η2221 | 1.15803 | 1.08022 | 0.95958 | 0.8762 | 0.83254 |
|  | σ | 0.0143 | 0.0163 | 0.0081 | 0.0139 | 0.0075 |
| (x) n-heptane + (1-x) iso-propanol |  |  |  |  |  |  |
| Grunberg-Nissan | d | -1.97241 | -1.91852 | -1.80002 | -1.7713 | -1.68887 |
|  | σ | 0.0352 | 0.0422 | 0.0386 | 0.0467 | 0.039 |
| Heric-Brewer | α12 | -1.20515 | -1.17167 | -1.13303 | -1.14173 | -1.07688 |
|  | α12 | -1.84051 | -1.78619 | -1.66795 | -1.63868 | -1.55674 |
|  |  | 0.42013 | 0.51062 | 0.45421 | 0.57211 | 0.46658 |
|  | σ | 0.0102 | 0.0115 | 0.0136 | 0.0113 | 0.0130 |
| Jouyban-Acree | A0 | -559.4203 | -555.95426 | -524.27459 | -530.98393 | -506.14069 |
|  | A1 | 90.51183 | 101.91733 | 127.37596 | 133.57252 | 129.64263 |
|  | A2 | -59.39539 | -40.92224 | -83.51906 | -36.70979 | -96.64779 |
|  | A3 | 110.94436 | 163.74655 | 43.07002 | 140.52597 | 61.97205 |
|  | σ | 0.0045 | 0.0064 | 0.0109 | 0.0087 | 0.0073 |
| McAllister three body model | η12 | 0.494 | 0.47786 | 0.44325 | 0.43256 | 0.39587 |
|  | η21 | 0.68497 | 0.60645 | 0.56619 | 0.49471 | 0.47211 |
|  | σ | 0.0102 | 0.0116 | 0.0136 | 0.0113 | 0.0130 |
| McAllister four body model | η1112 | 0.45932 | 0.44854 | 0.40454 | 0.40695 | 0.36044 |
|  | η1122 | 0.62563 | 0.56439 | 0.55435 | 0.48165 | 0.48448 |
|  | η2221 | 0.92647 | 0.82876 | 0.73406 | 0.66329 | 0.59372 |
|  | σ | 0.0079 | 0.0114 | 0.0102 | 0.0113 | 0.0075 |

Thermodynamic functions of activation

The energies of activation of viscous flow for the binaries studied were calculated using the following Eq. (21): 70

η=hNVexpΔGRT 	(21)

where η is viscosity of a liquid mixtures, h is Planck`s constant, N  is Avogadro`s number, V is the molar volume of the solution, R is general gas constant, T is temperature and G is the molar Gibbs energy of activation for the viscous flow process. Combining with

ΔG=ΔH−TΔS                                                          (22)

yields the equation

lnηVhN=ΔHRT−ΔSR 	(23)

where ΔHand ΔS are enthalpy and entropy of activation of viscous flow.

The plots of lnηVhN vs 1/T were found to be linear in the temperature range 288.15 to 308.15 K, as such the values of ΔH and ΔSwere obtained by the corresponding slopes and the intercepts. With ΔH and ΔS values as input in Eq. 22, the corresponding values of ΔG were also calculated. The values of thermodynamic functions of activation of viscous flow and of correlation coefficient (r) are listed in Table IX as a function of composition. The experimental values of ΔG against mole fraction are presented in Figs. S-5 - S-7 of the Supplementary material.

TABLE IX. Values of ΔG, ΔH, ΔSand r for the binary mixtures

| x | ΔG / J mol-1 |  |  |  |  | ΔH / J mol-1 | ΔS/ J mol-1K-1 | r |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T / K |  |  |  |  |  |  |  |
|  | 288.15 | 293.15 | 298.15 | 303.15 | 308.15 |  |  |  |
| (x) n-heptane + (1-x) ethanol |  |  |  |  |  |  |  |  |
| 0.0000 | 12660.30 | 12644.09 | 12627.88 | 12611.67 | 12595.46 | 13594.50 | 3.24 | 0.9992 |
| 0.1005 | 12461.31 | 12461.22 | 12461.13 | 12461.05 | 12460.96 | 12466.34 | 0.02 | 0.9978 |
| 0.2020 | 12326.33 | 12340.18 | 12354.04 | 12367.89 | 12381.74 | 11528.04 | -2.77 | 0.9971 |
| 0.3022 | 12245.13 | 12272.84 | 12300.54 | 12328.25 | 12355.95 | 10648.58 | -5.54 | 0.9984 |
| 0.4016 | 12162.76 | 12197.21 | 12231.67 | 12266.12 | 12300.57 | 10177.15 | -6.89 | 0.9983 |
| 0.5079 | 12094.13 | 12136.69 | 12179.26 | 12221.82 | 12264.39 | 9641.12 | -8.51 | 0.9933 |
| 0.5967 | 12047.06 | 12106.89 | 12166.72 | 12226.54 | 12286.37 | 8599.30 | -11.96 | 0.9984 |
| 0.7041 | 12000.25 | 12074.95 | 12149.66 | 12224.36 | 12299.07 | 7694.94 | -14.94 | 0.9960 |
| 0.8008 | 12004.89 | 12084.22 | 12163.54 | 12242.87 | 12322.19 | 7433.39 | -15.86 | 0.9890 |
| 0.8935 | 12053.60 | 12145.30 | 12236.99 | 12328.69 | 12420.39 | 6769.07 | -18.34 | 0.9992 |
| 1.0000 | 12178.33 | 12269.74 | 12361.14 | 12452.55 | 12543.95 | 6910.59 | -18.28 | 0.9948 |

TABLE IX. continued

| x | ΔG / J mol-1 |  |  |  |  | ΔH / J mol-1 | ΔS/ J mol-1K-1 | r |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | T / K |  |  |  |  |  |  |  |  |
|  | 288.15 | 293.15 | 298.15 | 303.15 | 308.15 |  |  |  |  |
| (x) n-heptane + (1-x) n-propanol |  |  |  |  |  |  |  |  |  |
| 0.0000 | 14730.52 | 14696.83 | 14663.13 | 14.629.43 | 14595.74 | 16672.46 | 6.74 |  | 0.9970 |
| 0.1020 | 14194.50 | 14181.02 | 14167.54 | 14154.06 | 14140.58 | 14971.27 | 2.69 |  | 0.9986 |
| 0.2018 | 13795.95 | 13798.36 | 13800.77 | 13803.18 | 13805.59 | 13657.12 | -0.48 |  | 0.9979 |
| 0.2990 | 13466.32 | 13470.58 | 13474.84 | 13479.11 | 13483.37 | 13220.72 | -0.85 |  | 0.9927 |
| 0.4005 | 13103.01 | 13119.76 | 13136.51 | 13153.26 | 13170.01 | 12137.60 | -3.35 |  | 0.9985 |
| 0.5017 | 12820.76 | 12845.94 | 12871.12 | 12896.30 | 12921.47 | 11369.77 | -5.03 |  | 0.9990 |
| 0.6033 | 12531.08 | 12580.81 | 12630.53 | 12680.26 | 12729.99 | 9665.25 | -9.94 |  | 0.9954 |
| 0.7025 | 12318.84 | 12394.59 | 12470.33 | 12546.08 | 12621.82 | 7953.70 | -15.15 |  | 0.9946 |
| 0.8010 | 12242.40 | 12324.62 | 12406.83 | 12489.05 | 12571.27 | 7504.20 | -16.44 |  | 0.9813 |
| 0.8918 | 12214.68 | 12294.00 | 12373.33 | 12452.65 | 12531.97 | 7643.22 | -15.86 |  | 0.9917 |
| 1.0000 | 12178.33 | 12269.74 | 12361.14 | 12452.55 | 12543.95 | 6910.59 | -18.28 |  | 0.9948 |
| (x) n-heptane + (1-x) iso-propanol |  |  |  |  |  |  |  |  |  |
| 0.0000 | 15018.63 | 14924.62 | 14830.61 | 14736.60 | 14642.59 | 20436.51 | 18.80 | 0.9993 |  |
| 0.1003 | 14234.64 | 14166.02 | 14097.40 | 14028.79 | 13960.17 | 18189.04 | 13.72 | 0.9993 |  |
| 0.2009 | 13664.96 | 13619.54 | 13574.12 | 13528.70 | 13483.28 | 16282.43 | 9.08 | 0.9990 |  |
| 0.3009 | 13210.61 | 13186.93 | 13163.25 | 13139.56 | 13115.88 | 14575.30 | 4.74 | 0.9971 |  |
| 0.3980 | 12854.01 | 12862.86 | 12871.70 | 12880.54 | 12889.39 | 12344.36 | -1.77 | 0.9970 |  |
| 0.5002 | 12566.97 | 12595.02 | 12623.07 | 12651.12 | 12679.16 | 10950.53 | -5.61 | 0.9923 |  |
| 0.5961 | 12347.29 | 12393.98 | 12440.66 | 12487.34 | 12534.02 | 9656.94 | -9.34 | 0.9927 |  |
| 0.6981 | 12223.10 | 12291.17 | 12359.24 | 12427.31 | 12495.38 | 8300.32 | -13.61 | 0.9994 |  |
| 0.8014 | 12172.33 | 12248.78 | 12325.23 | 12401.67 | 12478.12 | 7766.80 | -15.29 | 0.9735 |  |
| 0.9084 | 12156.33 | 12239.05 | 12321.77 | 12404.48 | 12487.20 | 7389.39 | -16.54 | 0.9784 |  |
| 1.0000 | 12178.33 | 12269.74 | 12361.14 | 12452.55 | 12543.95 | 6910.59 | -18.28 | 0.9948 |  |

The values of ΔH are more than two times higher for ethanol and n-propanol than for n-heptane, while for iso-propanol the values are more than three times higher than that for n-heptane, indicating that association and dipole-dipole interactions increase the value of  ΔH.

The value of S is negative for n-heptane and positive for ethanol, n-propanol and iso-propanol and shows that overall molecular order due to activated complex formation increases for n-heptane (nonassociating component) but decreases in case of ethanol, n-propanol and iso-propanol. The positive S for all alcohols indicated that, probably rupturing of hydrogen bonds formed through OH groups of alcohols in the activation process for viscous flow occurs, resulting in the structural disorder.71 The values of H and G are positive for all the binary mixtures and at all temperatures used. These values of free energy and enthalpy of activation of viscous flow increase with the alcohol concentration of solution at temperature constant. The values of G at constant concentration decrease if the temperature increase in accord with viscosity variation except for the mixtures concentrated in alcohols. The values of S are negative for all binary mixtures except for the mixtures concentrated in alcohols. These values show that overall molecular order due to activated complex formation increases for all binary mixtures except for the mixtures concentrated in alcohols.72

CONCLUSIONS

The densities and viscosities of binary mixtures of n-heptane with alcohols (ethanol, n-propanol and iso-propanol) were measured experimentally at temperatures between 288.15 and 308.15 K over the entire composition range. From these results, the excess values of molar volume and viscosity have been computed and fitted to the Redlich-Kister Equation. Positive deviations were observed for excess molar volumes and negative deviations for the excess dynamic viscosity at all of the temperatures and in the whole concentration range. Experimental results of the molar excess volume have been used to test the applicability of the Flory and Prigogine-Flory-Patterson theories. The values of percentage absolute average deviation obtained suggest the validity of these theories. Grunberg-Nissan, Heric-Brewer, Jouyban Acree, three-body and four-body McAlister models have been used to calculate viscosity coefficients and these were compared with experimental data for the mixtures. The results of these correlations showed that Jouyban Acree and four-body McAllister models are suitable to describe viscosities of the binary mixtures studied in this work.

The energies of activation of viscous flow for these binary mixtures were also calculated and discussed.

## REFERENCES

1. D. Gomez-Diz, J.C. Mejuto, J.M. Navaza, A. Rodriguez-Alvarez, J. Chem. Eng. Data 47 (2002) 872

2. J.A. Estrada-Baltazar, F.J. Juan, A. Gustavo,  J. Chem. Eng. Data 43 (1998) 441

3. H. Iloukhani, M. Jafari, Monatsh. Chem. 142 (2011) 1005

4. P.J. Flory, J. Am. Chem. Soc. 87 (1965) 1833

5. A. Abe, P.J. Flory, J. Am. Chem. Soc. 87 (1965) 1838

6. P.J. Flory, R.A. Orwoll, A. Vrij, J. Am. Chem. Soc. 86 (1964) 3507

7. R.A. Orwoll, P.J. Flory, J. Am. Chem. Soc. 89 (1967) 6814

8. H.T. Van, D. Patterson, J. Solution Chem. 11 (1982) 793

9. I. Prigogine, N. Trappeniers, V. Mathot, J. Chem. Phys. 21 (1953) 559

10. M.E. Wieser, Pure Appl. Chem. 78 (2006) 2051

11. F. Kohlrausch, W. Hallwachs, Wied. Ann d Phys. 50 (1893) 118

12. V. Dumitrescu, D. Sandulescu, Rev. Roum. Chim. 3 (1998) 183

13. A. Weissberger, Physical methods of Organic Chemistry Interscience Publishers Inc, New York (1959)

14. E. Jimenez, C. Franjo, L. Segade, J.L. Legido, M.I. Paz Andrade, Journal of Solution Chemistry 27 (1998) 569

15. M. Keller, S. Schnabel, A.  Heintz, Fluid Phase Equilib. 110 (1995) 231

16. R. Tanaka, S. Toyama, J. Chem. Thermodynamics 28 (1996) 1403

17. A.B. Pereiro, A. Rodriguez, J. Chem. Thermodynamics 39 (2007) 1219

18. B. Orge, M. Iglesias, A. Rodriguez, J.M. Canosa, J. Tojo, Fluid Phase Equilib. 133 (1997) 213

19. D. Papaioannou, D. Ziakas, C. Panayiotou, J. Chem. Eng. Data 36 (1991) 35

20. C.K. Zeberg-Mikkelsen, G. Watson, A. Baylaucq, G. Galliero, C. Boned, Fluid Phase Equilib. 245 (2006) 6

21. J. Peleteiro, C.A. Tovar, R. Escudero, E. Carballo, J.L. Legido, L. Romani, Journal of Solution  Chemistry 22 (1993) 1005

22. T.R.C. Thermodynamic Tables-Hydrocarbons, Thermodynamics Research Center, The Texas A & M. University Sistem: College Station, TX (1993)

23. L. Fernandez, E. Perez, J. Ortega, J. Canosa, J. Wisniak, Fluid Phase Equilib. 341 (2013) 105

24. T.M. Aminabhavi, V.B. Patil, M.I. Aralaguppi, H.T.S. Phayde, J. Chem. Eng. Data 41 (1996) 521

25. N.V. Sastry, M.M. Raj,  J. Chem. Eng. Data 41 (1996) 612

26. T.R.C. Thermodynamic Tables Hydrocarbons Supplement 92, Thermodynamic Research Center, Texas A & M. University College Station, Texas A & M. University TX (1988)

27. T.E. Daubert, R.P. Danner, Physical and Thermodynamic Properties of Pure Chemicals: Data Compilation (Hemisphere Publishing Corporation 1989)

28. M.L. Kijevcanin, S.P. Serbanovic, I.R. Radovic, B.D. Djordjevic, A.Z. Tasic, Fluid Phase Equilib. 251 (2007) 78

29. A. Rodriguez, J. Canosa, A. Dominquez, J. Tojo, Fluid Phase Equilib. 216 (2004) 167

30. G. Garcia-Miaja, J. Troncoso, L. Romani, Fluid Phase Equilib. 274 (2008) 59

31. H.E.  Hoga, R.B. Torres, J. Chem. Thermodynamics 43 (2011) 1104

32. P.S. Nikam, M.C. Jadhav, M. Hasan, J. Chem. Eng. Data 40 (1995) 931

33. F.M. Pang, C.E. Seng, T.T. Teng, M.H. Ibrahim, J. Mol. Liq. 136 (2007) 71

34. Z. Shan, A.A. Asfour, Fluid Phase Equilib. 143 (1998) 253

35. A. Chafer, J. de la Torre, E. Lladosa, J.B. Monton, Fluid Phase Equilib. 361 (2014) 23

36. J.A. Riddick, W.B. Bunger, T.K. Sakano, Organic Solvents. Physical Properties and Methods of Purification Techniques of Chemistry, 4th Ed. (Wiley Interscience: New York 1986)

37. S.L. Oswal, H.S. Desai, Fluid Phase Equilib. 149 (1998) 359

38. M.G. Prolongo, R.M. Masegosa, H.I. Fuentes, A. Horta, J. Phys. Chem. 88 (1984) 2163

39. G.P. Dubey, M. Sharma, N. Dubey, J. Chem. Thermodynamics 40 (2008) 309

40. C. Franjo, E. Jimenez, T.P. Iglesias, J.L. Legido, M.I. Paz Andrade, J. Chem. Eng. Data 40 (1995) 68

41. C. Franjo, C.P. Menaut, E. Jimenez, J.L. Legido, M.I. Paz Andrade, J. Chem. Eng. Data 40 (1995) 992

42. L. Romani, J. Peleteiro, T.P. Iglesias, E. Carballo, R. Escudero, J.L. Legido, J. Chem. Eng. Data 39 (1994) 19

43. J. Ortega, M.I. Paz Andrade, E. Rodriguez, J. Chem. Eng. Data 31 (1986) 336

44. R. Mehra, M. Pancholi, Indian Journal of Physics 80 (2006) 253

45. C. Yang, W. Xu, P. Ma, J. Chem. Eng. Data 49 (2004) 1794

46. M.N. Roy, A. Sinha, B. Sinha, J. Solution Chem. 34 (2005) 1311

47. A.C. Gomez-Marigliano, A. Arce, E. Rodil, A. Soto, J. Chem. Eng. Data 55 (2010) 92

48. S. Aznarez, M.M.E.F.D.R. Holgado, E.L. Arancibia, J. Mol. Liq. 124 (2006) 78

49. P.S. Nikam, B.S. Jagdale, A.B. Sawant, M. Hasan, J. Chem. Eng. Data 45 (2000) 214

50. A.R. Mahajan, S.R. Mirgane, Journal of Thermodynamics (2013) doi:10.1155/2013/571918

51. D. Săndulescu, Chimie Fizică 1rd edn. (Ed. Ştiinţifică, Bucureşti 1979)

52. T.M. Letcher, R.C. Baxter, Journal of Solution Chemistry 18 (1989) 65

53. H. Funke, M. Wetzel, A. Heintz, Pure Appl. Chem. 61 (1989) 1429

54. R.B. Torres, A.Z. Francesconi, P.L.O. Volpe, Fluid Phase Equilib. 210 (2003) 287

55. A.J. Treszczanowicz, D. Patterson, G.C. Benson, T. Guttman-Kasprzycka, Fluid Phase Equilib. 50 (1989) 235

56. A. Heintz, Ber Bunsenges Phys. Chem. 89 (1985) 172

57. D. Kulikov, S.P. Verevkin, A. Heintz, Fluid Phase Equilib. 192 (2001) 187

58. E. Bich, D. Papaioannou, A. Heintz, E. Tusel-Langer, R.N. Lichtenthaler, Fluid Phase Equilib. 156 (1999) 115

59. U. Domanska, Fluid Phase Equilib. 130 (1997) 207

60. M. Bender, A. Heintz, Fluid Phase Equilib. 89 (1993) 197

61. J.B. Irving, NEL Report 630 National Eng Lab East Kilbride, UK (1977)

62. J.B. Irving, NEL Report 631 National Eng Lab East Kilbride, UK (1977)

63. L. Grunberg, A.H. Nissan, Nature 164 (1949) 799

64. E.L. Heric, J.G. Brewer, J. Chem. Eng. Data 12 (1967) 574

65. A. Jouyban, M. Khoubnasabjafari, Z. Vaez-Gharamaleki, Z. Fekari, W.E. Jr. Acree, J. Chem. Pharm. Bull 53 (2005) 519

66. A. Jouyban, A. Fathi-Azarbayjani, M. Khoubnasabjafari, W.E. Jr. Acree, Ind. J. Chem. 44 (2005) 1553

67. R.A. McAllister, AIChE J. 6 (1960) 427

68. D.W. Marquardt, J. Soc. Indust. Appl. Math. 11 (1963) 431

69. R.K. Nigam, B.S. Mahl, Indian Journal of Chemistry 9 (1971) 1255

70. H. Eyring, M.S. Jhon, Significant Liquid Structures (John Wiley, New York 1969)

71. M.A. Saleh, S. Akhtar, S. Begum, M.S. Ahmed, S.K. Begum, Physics and Chemistry of Liquids 42 (2004) 615

72. M.E.F. De Ruiz Holgado, C.R. De Schaefer, F. Davolio, M. Katz, Can. J. Chem. 71 (1993) 790.